Twin Oaks

Hattiesburg, Mississippi

1) Date of Inspection:
February 25, 2004

Twin Oaks

2) Bldg. #10
144 Units/12 Apartments

3) Remove all water from floors, spray bleach for mold

4) Caulk all holes penetrating wood (top plates – firewalls)

5) Firecaulk all sheetrock at roof decks for firewalls and smokewalls

6) Seal sheetrock in firewalls and smoke walls where penetrations occur

7) Install fire dampers in bathroom vents

8) Complete all electrical work

9) Complete porches

10) Complete all siding

11) Paint hand rails on stairs.

12) Complete installation of A/C units

13) Install smoke alarms

14) Complete drain lines for hot water heaters

15) Close holes under tubs

16) Complete all soffit under stairs

17) Secure all strapping for plumbing

18) Verify can light in kitchen – third floor is fire rated

19) Secure plastic plumbing lines on third floor

20) Hang hat channel on ceilings (install sheet rock)

___________________________
Date:
____________________


Chuck Dammon

21)          VP/Project Manager

