Wall Repairs & Painting

Slidell City Court

REQUEST FOR PROPOSALS

Slidell City Court sustained hurricane damages that require the services of a contractor to complete the work to repair walls where wall covering (paper) has been removed, tape, float, texture and paint the walls.
Replace vinyl base and clean up the area on completion of the task.

The area requiring this work is approximately 1,000 s.f. or less, in the foyer and hallways to the administrative office at the Slidell Court Building 501 Bouscaren, Slidell, Louisiana.

Final acceptance and payment will be made by the Slidell City Court upon final completion and acceptance of all work, including clean up of all areas disturbed by the work, and removal from the premises of all construction debris and materials.

Work may be accomplished on certain days when the court is not in session, or on weekends, the schedule of which is to be worked out between the successful contractor and Judge Jim Lamz’s staff.

Proposal Due Date:
July 24, 2006

Submitted to:
Dammon Engineering, Inc.

1095 Florida Ave.


Slidell, Louisiana 70458

Attached are general specifications for the work.

PAINTING

1 PART 1 GENERAL


.1

SECTION INCLUDES



A.
Surface preparation and field application of paints and coatings.


.2
seq level2 \h \r0 
REFERENCES



A.
ASTM D16 ‑ Definitions of Terms Relating to Paint, Varnish, Lacquer, and Related Products.



B.
NPCA (National Paint and Coatings Association) ‑ Guide to U.S. Government Paint Specifications.



C.
PDCA (Painting and Decorating Contractors of America) -Painting ‑ Architectural Specifications Manual.


.3seq level2 \h \r0 

SUBMITTALS



A.
Product Data:  Provide data on all finishing products.



B.
Samples:
Provide samples of each color and materials to be applied with texture to simulate actual conditions on representative samples of the actual substrate.



C.
Manufacturer's Instructions:
Indicate special surface preparation procedures, substrate conditions requiring special attention.


.4seq level2 \h \r0 

QUALIFICATIONS



A.
Manufacturer:  Company specializing in manufacturing the Products specified in this section with minimum ten years documented experience.



B.
Applicator:  Company specializing in performing the work of this section with minimum 3 years-documented experience.


.5seq level2 \h \r0 
 
REGULATORY REQUIREMENTS



A.
Conform to applicable code for flame and smoke rating requirements for finishes.


.6seq level2 \h \r0 

FIELD SAMPLES



A.
Accepted sample may remain as part of the Work.


.7seq level2 \h \r0 

DELIVERY, STORAGE, AND HANDLING





A.
Deliver products to site in sealed and labeled containers; inspect to verify acceptability.



B.
Container label to include manufacturer's name, type of paint, brand name, lot number, brand code, coverage, surface preparation, drying time, cleanup requirements, color designation, and instructions for mixing and reducing.



C.
Store paint materials at minimum ambient temperature of 45 degrees F and a maximum of 90 degrees F, in ventilated area, and as required by manufacturer's instructions.


.8
seq level2 \h \r0 
ENVIRONMENTAL REQUIREMENTS



A.
Do not apply materials when surface and ambient temperatures are outside the temperature ranges required by the paint product manufacturer.



B.
Minimum Application Temperatures for Latex Paints:  45 degrees F for interiors; unless required otherwise by manufacturer's instructions.



C.
Provide lighting level of 80-ft candles measured mid‑height at substrate surface.

2

seq level1 \h \r0 

seq level2 \h \r0  PART 2 PRODUCTS


.1

MANUFACTURERS



A.
Manufacturers ‑ Paint




1.
Sherwin Williams




2.
Devo             




3.
Benjamin Moore  



4.
Acceptable Equivalent Substitute 


.2

seq level2 \h \r0 

MATERIALS



A.
Primers: Latex based interior white primer used on interior gypsum drywall under a flat latex paint.



B.
Coatings:  Ready mixed, latex based paint for use as a flat finish over prime coated gypsum drywall.  Process pigments to a soft paste consistency, capable of being readily and uniformly dispersed to a homogeneous coating; good flow and brushing properties; capable of drying or curing free of streaks or sags.



C.
Accessory Materials:
Linseed oil, shellac, turpentine, paint thinners and other materials not specifically indicated but required to achieve the finishes specified, of commercial quality.



D.
Patching Materials:  Latex filler.



E.
Fastener Head Cover Materials:
Latex filler.


.3

seq level2 \h \r0 

FINISHES



A.
Refer to schedule at end of section for surface finish and color schedule.

3

seq level1 \h \r0 

seq level2 \h \r0  PART 3 EXECUTION


.1

EXAMINATION



A.
Verify site conditions.



B.
Verify that surfaces are ready to receive work as instructed by the product manufacturer.



C.
Examine surfaces scheduled to be finished prior to commencement of work.  Report any condition that may potentially affect proper application.



D.
Measure moisture content of surfaces using an electronic moisture meter.  Do not apply finishes unless moisture content of surfaces are below the following maximums:




1.
Plaster and Gypsum Wallboard:  12 percent.




2.
Masonry, Concrete, and Concrete Unit Masonry:  12 percent.




3.
Interior Wood:  15 percent, measured in accordance with ASTM D2016.


.2

seq level2 \h \r0 

seq level3 \h \r0 

 PREPARATION



A.
Remove or mask doors and trim, electrical plates, hardware,  escutcheons, and fittings prior to preparing surfaces or finishing.  Store for reinstallation.


B.
Correct defects and clean surfaces which affect work of this section.  Remove existing materials and/or coatings that exhibit loose surface defects.



C.
Seal with shellac and seal marks which may bleed through surface finishes.



D.
Impervious Surfaces:
Remove mildew by scrubbing with solution of tri‑sodium phosphate and bleach.  Rinse with clean water and allow surface to dry.



E.
Gypsum Board Surfaces:  Fill defects with filler compound. Sand as required and spot prime defects after repair.


.3

seq level2 \h \r0 

APPLICATION



A.
Apply products in accordance with manufacturer's instructions.



B.
Do not apply finishes to surfaces that are not dry.



C.
Apply each coat to uniform finish and color.


D.
Vacuum clean surfaces free of loose particles.  Use tack cloth just prior to applying next coat.



E.
Allow applied coat to dry before next coat is applied.


.4

seq level2 \h \r0 

FINISHING MECHANICAL AND ELECTRICAL EQUIPMENT



A.
Reinstall electrical cover plates, hardware, light fixture trim, escutcheons, and fittings removed prior to finishing.


.5
seq level2 \h \r0 
FIELD QUALITY CONTROL



A.
Field inspection will be provided by Dammon Engineering, Inc.

.6

seq level2 \h \r0 

CLEANING



A.
Clean work thoroughly.



B.
Collect waste material, which may constitute a fire hazard, place in closed metal containers and remove daily from site.


.7seq level2 \h \r0 

seq level3 \h \r0 

SCHEDULE ‑ INTERIOR SURFACES



A.
Wood ‑ Painted:




1.
One coat of Sherman Williams or Benjamin prime sealer.




2.
Two coats of alkyd enamel, gloss.    


.8seq level2 \h \r0 

seq level3 \h \r0 

SCHEDULE ‑ COLORS



A.
Match existing colors for all new surfaces and with touch up.  (All colors shall be approved by Judge James Lamz.
Match existing colors on walls.  Assure that patching has been accomplished prior to painting.


END OF SECTION
PAGE  
Slidell City Court Page 1 of 5

