March 6, 2003

Mr. Ali Abedali

2309 Elizabeth Street

Metairie, Louisiana  70003

RE:
Lot 11, Sq. 5, Pontchartrain Shores


Jefferson Parish Permit #1334-02

Dear Mr. Abedali,

With regard to our meeting at my office this morning, please note the following:

1. The pilings that are off center in the beam on the garage wall can be used as is, except that two additional No. 5 bars should be run along the centerline of piles and then bent into line with the center of the beam.  Dig out at least one foot around the piles for additional concrete support, prior to pouring concrete.

2. Since the piles were driven in the wrong location at the right rear of the house, they can be used as is by adding to the length of the floor trusses for the second floor and roof joist above the second floor, using the correct span and spacings.

3. The existing wire mesh that is spanning the foundation in the wrong direction is acceptable, as there are concrete beams that intercept the span sufficiently to prevent a problem in temperature expansion.

4. The actual size of the concrete slab beams was increased to 20” x 24”.  This is approved.

If you have any questions, or require any further information, please feel free to call.

Sincerely,

Edwin J. deMeritt, NCARB

