New Orleans Zoning Ordinance

4.6.7. Height, Area and Yard Requirements.

The minimum requirements for lot area, yards and maximum height in the RD-3 Two-Family Residential District are contained Table 4.F. These standards apply to all permitted and accessory uses, unless a variance is granted by the Board of Zoning Adjustments under Section 14.6, and to all conditional uses unless modified by the City Council in conjunction with approval of a conditional use permit under Section 16.6.

Table 4.F
Area Regulations for the RD-3 Two-Family Residential District
	Requirements
	Standards: Type of Development: One-Family
	Standards: Type of Development: Two-Family1
	Standards: Type of Development: Town House1
	Standards: Type of Development: Nonresidential

	Minimum lot area per dwelling unit or nonresidential site area 
	3,600 sq. ft.
	1,800 sq. ft.
	2,000 sq. ft.
	20,000 sq. ft.

	Minimum lot width 
	30 ft.
	30 ft.
	18 ft.
	100 ft.

	Minimum lot depth 
	90 ft.
	90 ft.
	90 ft.
	100 ft.

	Maximum height 
	40 ft.
	40 ft.
	40 ft.
	40 ft.

	Minimum depth of front yard2
	20 ft.
	20 ft.
	20 ft.
	20 ft.

	Minimum aggregate width of side yards(Percent of actual lot width)
	20%
	20%
	See Note 1
	20%

	Minimum width of each side yard 
	3 ft.
	3 ft.
	10 ft.
	10 ft.

	Maximum required aggregate width of side yards
	12 ft.
	12 ft.
	20 ft.
	20 ft.

	Minimum depth of rear yard 
	20 ft.
	20 ft.
	20 ft.
	20 ft.


Table 4.F Notes:

1 See Article 11 for supplemental regulations governing two-family dwellings and town houses.

2 For a corner lot, a minimum yard of ten (10) feet shall be provided on the side street.

