DAMMON ENGINEERING, INC.

dammonengineering.com

CONSULTING		DESIGN		STUDIES		EXPERT WITNESS

554 Old Spanish Trail			                 				     985-649-5832

Slidell, LA 70458				  			            FAX 985-641-5950

## March 12, 2012

To: 	Jay Pittman, E.I.

St. Tammany Parish

Department of Engineering

21490 Koop Dr.

Mandeville, LA 70471

REF: Living the Word International Church

2528 Old Spanish Trail

Slidell, LA

Dear Mr. Pittman,

Here are the cut and fill calculations for this site. The crossed hatch area on the dwg represents a no work area. The shaded hatch area in the front represents the relocated pond. The non shaded areas in the parking area and swale area represent where cuts will be made to achieve new elevations. We have also lowered the parking areas by 6” and the swales to help increase the amount of fill that is being removed from the site.

Fill below B.F.E. for church pad= 1850 yd3

Cut below B.F.E. for the parking area = (35000ft2)(0.7ft) /27 = 907 yd3

Cut below the B.F.E. for the drainage swale (5394 ft2)(1.3ft) /27 = 259 yd3

Cut below the B.F.E. for the relocated detention pond (5920 ft2)(7.9ft/2) /27 = 867 yd3

Total cuts = 2033 yd3

Total fill   = 1850 yd3

SUM = 183 yd3 extra being removed from Site

The existing crushed limestone on the Site shall be used as the base material for the concrete paving. There shall be no additional fill required for the concrete paved vehicular areas.

The existing pond shall be reworked to allow the new parking area elevations and meet the minimum required cuts needed to achieve “No net Fill” for the Site.. The pond shall maintain its current load capacity. The relocated pond will be used for water storage although the new parking lot will retain its water prior to being released as designed.

Sincerely,

Emmett G. Dammon