STANDARD
7 Mar 2000
06202

UPDATES  -



SECTION  06202


DOOR, FRAME, AND FINISH HARDWARE INSTALLATION
PART 1 GENERAL
Edit Article ‘1.1' as necessary to give an accurate, but brief description of the work of this Section and accurate listing of related work, especially if the scope of the work is not shown graphically on the Drawings.

 1.1
SUMMARY
A.
Includes But Not Limited To

1.
Furnish and install sealants for calking door frames as described in Contract Documents.

B.
Products Installed But Not Supplied Under This Section

1.
Hollow metal doors

2.
Hollow metal door frames

3.
Finish hardware

C.
Related Sections

1.
Division 04  -  Grouting of frames installed in masonry walls
04814 04815
2.
Division 07  -  Quality of sealants
07920
3.
Division 08  -

a.
Furnishing of doors and metal frames
08112 08115 08215
b.
Furnishing of finish hardware
08701
 1.2
SUBMITTALS
A.
Quality Assurance / Control
Quality
1.
Inspection report verifying correct operation and adjustment of installed hardware.

2.
Copy of 'Installation Guide for Doors & Hardware' by Door & Hardware Institute. Guide may be obtained from Door and Hardware Institute (DHI).

 1.3
QUALITY ASSURANCE
A.
Regulatory Requirements  -  Fire door installations shall meet code requirements.

B.
Pre-Installation Conference
Pre-installation
1.
Schedule conference after hardware has been delivered to site and organized into hardware groups by door, but prior to installation of hardware.

2.
Check for appropriate blocking and for correct hardware models and fasteners for substrates.

3.
Review set of Manufacturer’s installation, adjustment, and maintenance instructions submitted under Section 08701.

Delete Article ‘1.4' if no wood doors used on Project.

 1.4
DELIVERY, STORAGE, AND HANDLING

Edit to fit Project key cabinet circumstances.

AA.A.
Keys  -  Immediately prior to Final Acceptance Meeting, turn change keys over to Owner properly organized and tagged.

 1.5
OWNER'S INSTRUCTIONS
A.
Explain keying systems at time keys and locking mechanisms are tested by using Owner's Operations And Maintenance Manual.

PART 2 PRODUCTS  -  Not Used

PART 3 EXECUTION
3.1
INSTALLATION
Installation
A.
Hollow Metal Frames

1.
Site Tolerances  -

a.
Squareness  -  1/16 inch from top edge to opposite top edge.

b.
Plumbness  -  1/16 inch from top of jamb to bottom of jamb.

c.
Alignment  -  1/16 inch from plane of left side face of jamb to right side face of jamb.

d.
Twist  -  1/16 inch across throat of jamb plane measured across each face to plane of opposite jamb throat.

e.
Finished Clearance Between Door And Frame  -

1)
1/16 inch at head and hinge jamb plus 1/16 maximum

2)
1/8 inch at strike jamb plus or minus 1/16 inch maximum.

3)
1/2 inch to top of finished floor surface or 1/4 inch to top of threshold, plus or minus 1/16 inch maximum.

2.
Set frame in location and level head.

3.
Equalize with adjustable floor anchor.

4.
Set spreaders and fasten jambs to floor and wall.

a.
Wood spreaders shall be square, fabricated from lumber one inch minimum thick, be same length as door opening at header, and same depth as frame.

b.
Cut notches for frame stops.

c.
Do not remove spreaders until frames are permanently anchored in wall.

d.
Use one spreader at base of frame and another at strike level.

e.
Do not use temporary spreaders welded to base of jambs during installation of frame.

5.
Fill gap between frame and framing with urethane foam or fiberglass insulation.  If urethane foam is used, coat interior of frames with foam prior to installing frame.  Trim excess before installation of frame.

6.
Calking  -

a.
Calk around both sides of frames of doors receiving acoustical seals with specified sealant.

Delete paragraph ‘b’ if no door frames installed in exposed masonry walls.

b.
Calk around both sides of frames installed in exposed masonry walls with specified sealant.

B.
Doors

1.
When Project is completed, doors shall not bind, stick, or be mounted so as to cause future hardware difficulties.

2.
Do not impair utility or structural strength of door in fitting of door, applying hardware, or cutting and altering door louvers, panels, or other special details.

C.
Hardware

1.
General  -

a.
Install using set of Manufacturer’s installation, adjustment, and maintenance instructions submitted with hardware under Section 08701.  Follow as closely as possible.

It is preferred that door closers be mounted as specified than to mount them out of corridors and in the room.

b.
Mount closers on jamb stop side of door (Parallel Arm) where it is physically possible to do so and not hinder operation of door or closer.

 3.2
FIELD QUALITY CONTROL
A.
Tests  -  Arrange to have keys brought to Project site and, in meeting attended by local representatives and Architect, test every new key and locking mechanism.


END OF SECTION

ADVANCE \d252
B L A N K   P A G E

557-1804 (20020)
3
06202

Orlando Welfare Complex
October 11, 2000
Door, Frame, and 



Finish Hardware Installation


