SECTION 11160

LOADING DOCK EQUIPMENT

PART 1 GENERAL 

1.1 SUMMARY 

A. Products supplied but not installed under this section.

1. Dock bumpers as described in contract documents.

2. Dock levelers as described in contract documents.

3. Dock shelter/seals as described in contract documents. 

B. Related sections 

1. Divisions 02 and 05 – Pipe bollards. 

2. Division 06-installation. 

1.2 SUBMITTALS 

A. See Section 01300.

B. Product data – manufacturer’s literature or cut sheet including installation and maintenance instructions.  

PART 2 PRODUCTS 

2.1 MATERIALS 

A. Laminated tread bumpers. 

1. Size indicated – as shown on drawings.  

2. Fabricated from multiple plies cut from fabric-reinforced rubber tires to uniform thickness of 9”.  

3. Laminate plies under pressure on ¾” diameter steel supporting rods that are welded and bolted to 1/4 “ thick structural closure steel angles with predrilled anchor holes.  

4. Size angles to provide not less that 1” of tread plies extending beyond the face of closure angles.  

5. Bumpers must be adequate to accommodate 5% grade of bay.  

B. Dock leveler

1. Model:  Rite-Hite HD3187.

2. 7’-0” wide x 8’-0” long.

3. Hydraulically operated.

4. 35,000 lbs. Capacity. 

5. Operating range 12” above and 12” below dock level.

6. 20” lip shall extend 16” beyond 9” thick bumper.

7. 7” high Safe-T-Lip barrier.

8. Maintenance strut.

9. Control Station:  single button NEMA 12,208 volt, 3 phase.

10. Dock leveler to be complete packaged unit including:  

a. Motor – 208 volt, 60HZ, 3 phase.

b. Pump.

c. Valve assembly power pack.

d. Night locks.

e. Maintenance strut.

f. Full operating range toe guard.

g. Laminated bumpers.

C. Dock shelter/seals

1. For the purpose of establishing criteria as a basic for bidding, the following dock shelter/seal by Rite-Hite is described:  

a. Model:  Frommelt Eliminator Gapmaster 600.  

b. Size:  12’ wide x 10’-0” high x 21” projection at top and 26” projection at bottom.  

c. Removable side curtains attached to impactable foam side frames with Velcro.  

d. Head curtain adjustable with pull ropes and splits to be 30”.  

e. Side and head curtain fabric:  40 oz. vinyl.  

f. Fabric color:  black.  

2.2 APPROVED BUMPER MANUFACTURERS

A. Durable Corporation, Norwalk, OH  (800) 537-1603 FAX (800) 537-6287.  

B. Kelly company, Inc., Milwaukee, WI  (800) 558-2168 FAX (414) 352-2093.  

C. Pawling Corporation, Wassaic, NY  (914) 373-9300.  

D. Pioneer Manufacturing, Inc., Spring Hill, TN  (800) 251-2382

FAX (615) 486-0316.

2.3 APPROVED DOCK LEVELER MANUFACTURERS 

A. Rite-Hite Corporation, Milwaukee, WI  (800) 365-7735 FAX (414) 355-9248. 

B. W.B. McGuire Co., Inc., Hudson, NY  (800) 6240-8473 TAX (518) 828-1262.

C. Kelly Company, Inc., Milwaukee, WI  (800) 558-2166 TAX (414) 352-2093.

D. Serco, Dallas, TX  (214) 33-4111.  

PART 3 EXECUTION 

3.1 INSTALLATION 

A. Install dock bumpers and dock leveler in accordance with manufacturer’s written directions and as shown on the drawings.  

B. Attach dock bumpers to structure so they comply with requirements indicated for spacing, arrangement, and position relative to top of platform and driveway surface.  

END OF SECTION

557-1804 (20020) 

Section 11160

Orlando Welfare Complex
1
Loading Dock Equipment 
557-1804 (20020)
2
Section 11160

Orlando Welfare Complex
October 11, 2000
Loading Dock Equipment

