STANDARD
5 Aug 1999
12495

UPDATES  -

16 Jul 98 -
Added Fantagraph fabrics as an A & R option.



SECTION  12495


CURTAINS AND DRAPES
PART 1 GENERAL
 1.1
SUMMARY
A.
Includes But Not Limited To

1.
Furnish and install draperies, curtains, and hardware as described in Contract Documents.

 1.2
REFERENCES
A.
American National Standards Institute / National Fire Protection Association
ANSI
1.
ANSI / NFPA 701, 'Fire Tests for Flame-Resistant Textiles and Films'

 1.3
SUBMITTALS
A.
Product Data
Product Data
1.
Fabric Supplier's literature or cut sheets on fabric

2.
Curtain Rod Manufacturer's literature or cut sheets

3.
Flameproofing literature

4.
Color and style selection

5.
Operating and maintenance instructions

B.
Samples
Samples
1.
24 inch wide and 48 inch high sample including all specified elements of finished curtains, including flame retardant certification tag.  Do not fabricate Project drapes until sample has been reviewed and approved by Architect.

2.
Submit sample with Product Data submittal.  Sample will serve as standard by which to evaluate Project curtains.

C.
Quality Assurance / Control  -  Certificate of flame-proofing
Quality
 1.4
QUALITY ASSURANCE
A.
Requirements of Regulatory Agencies  -  Material used shall be inherently flame retardant with a flame spread rating meeting code requirements when tested in accordance with ANSI / NFPA 701.  Attach permanent tag to each panel attesting to flame retardant quality of material used.

PART 2 PRODUCTS
 2.1
MANUFACTURED UNITS
A.
Fabric

OPTION ONE  -  New Projects and Addition / Remodel Projects using Church color schemes

Contact individual suppliers for memo samples to determine color selections to be specified.

1.
Approved Fabrics And Colors  -
Approved Products
Select either opaque or casement fabrics for the Project.  If both are desired, the locations for each should be specified or included in the drapery schedule on the Drawings.  Where colors are specified in the Master Specification, use the colors that correspond to the Church color scheme being used.  No other colors are authorized without written approval of A & E Services after submission of a color board.

a.
Off-White Opaque Drapery  -

1)
5th Avenue, Park Avenue, or Bourbon Street by Coral  -  Off-White, Ivory, or Cameo

2)
Cottonwood or Snowbird by InterSpec

a.
Tinted Opaque Drapery  -

1)
Fanshell by Interspec  -  17 Danube; 16 Spruce; 14 Petal

2)
Petals by InterSpec  -  17 Steel; 13 Seafoam; 46 Tearose

a.
Casements  -

1)
Handcart or Frontier by Carole

2)
Granger, Jordan, or Layton by Interspec

3)
Zanzibar #01 or Montpelier by Maharam

b.
Blackout Drapery  -

1)
Stoplight by Coral

2)
Control Line I by Maharam

3)
Control Line VII No. 600203 by Maharam

OPTION TWO  -  Addition / Remodel Projects where Church color schemes are not being used.

Contact individual suppliers for memo samples to determine color selections to be specified.

1.
Colors as selected by Architect.

2.
Approved Fabrics  -
Approved Products
a.
Casements  -

1)
Handcart or Frontier by Carole

2)
Granger, Jordan, or Layton by Interspec

3)
Zanzibar #01 by Maharam

b.
Opaques  -

1)
5th Avenue, Park Avenue, or Bourbon Street by Coral

2)
Fanshell by Interspec

3)
Cottonwood or Snowbird by InterSpec

4)
Petals by Interspec  -  3 Fawn, 15 Sand, 32 Dune, 13 Seafoam, 46 Tearose, 17 Steel

5)
Conova by Fantagraph  -  Canvas, Creme

6)
Caron by Fantagraph  -  Canvas, Frost

7)
Millhouse by Fantagraph  -  Canvas, Frost

c.
Blackout  -

1)
Stoplight by Coral

2)
Control Line I by Maharam

3)
Control Line VII No. 600203 by Maharam

Sheers are only authorized as replacements for existing sheer curtains.

d.
Sheers  -

1)
Exotica aka Flutter by Coral

3.
Approved Fabric Suppliers  -
Approved Products
Only the fabrics specified as obtained through the specified suppliers are acceptable.  Do not allow ‘knock‑offs’ from other suppliers to be used.

a.
Carole, Augusta, GA  (706) 863-4742  

b.
Coral Fabrics, Chicago, IL  (800) 621-5250  www.coralofchicago.com
c.
Fantagraph, Cincinnati, OH  (800) 888-5000  

d.
Interspec, Allenwood, NJ  (800) 526-2800  

e.
Maharam, Hauppage, NY  (800) 645-3943  

B.
Drapery Hooks  -  Stainless steel, standard 1-1/2 inch hook with pointed hook top.

C.
Drapery Rods

1.
Rods shall be sufficient width, window width plus 1/3, to allow drape to stack clear of window opening but no wider.  This requirement may be modified as follows  -

a.
Where Drawings detail differently.

b.
Where wall, cabinets, mechanical equipment, or other obstruction requires modification.

c.
Where symmetry of a room would indicate desirability of an exception.

2.
Traverse rods shall include wall or floor mounted tension pulleys for endless cord operation.

3.
Approved Models And Manufacturers  -
Approved Products
a.
'Super Fine' by Kirsch Co, Sturgis, MI  (800) 528-1407  www.kirsch.com
b.
Graber 601 Series by Springs Window Fashions Division Inc, Montgomery, PA  (800) 544-4749  www.swfd.com
 2.2
FABRICATION
A.
Double top and bottom hems unless specifically specified otherwise.

B.
Provide necessary weights.

C.
Fullness shall be minimum of 2-1/2 times width of space covered by drape.

D.
Space pleats 4 inches maximum center of pleat to center of pleat.

E.
Drapes shall have

1.
Fabric inspected over back-lite table for flaws

2.
Straight, even blindstitched side and bottom hems

3.
Seams hidden beside pleats

4.
Joined seams serged and overcast with no puckering

5.
4 inch double hems and headings

6.
2 inch overlap, total of 4 inches on pair

7.
Stackoff of 1/3 of window width

8.
Sampson permanent crinoline used in the heading

9.
Seams match up on bottom hems

10.
Corners of bottom hems closed with hand stitching

11.
Pleats evenly spaced to size

12.
Straight edge across top after pleating

13.
Straight, even folds

PART 3 EXECUTION
 3.1
INSTALLATION
Installation
A.
Install tracks, wall or ceiling mount, with mounting device head no larger than No. 6, to yield direct withdrawal strength of 25 lbs minimum.

B.
Support spacing to be as recommended in Manufacturer's literature.

 3.2
CLEANING
A.
Tracks to be free of marring, scratches, and foreign material.


END OF SECTION

20020
1
Section 12495

MeetingHouse

Curtains And Drapes


