Tub Room

- Water takes a LONG time to get luke warm and never reaches a “hot” temperature. Even the warm water doesn’t last long. Water pressure is insufficient as well – as seen by Fred Hayes

Men’s and Women’s Locker Rooms

- Water takes a very long time to reach a tolerable temperature and never reaches the optimal and comfortable temperature for showering. NEVER gets hot and what warm water there is dies away much too quickly. Pressure is VERY low, especially if other appliances are running (washers, dishwasher, Vichy etc – or even one of the sinks in the bathroom)

Vichy  Room

- Takes a very long time to change between hot and cold. Warm water dies away quickly.

- Sometimes the hot water will not work at all. Knobs difficult to make work also.

1ST make a direct connection from the hot water heater to the washing machine(s)

2nd make sure that the hot water loop extends all the way to above the tub room near the existing Temper’d Hot water Valve.

3rd Verify that all existing line(s) used in the hot water loop are true 1” diameter water line.   If so, replace the connectors currently used with an exterior coupling with no unnecessary elbows.

4th Insulate all hot water lines

5th