554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

May 30, 2018

Perry Watkins

Pontchartrain Materials Corp, LLC

3819 France Road

New Orleans, LA

Re:  Trailer Hurricane Tie Down(s) for PMC New Orleans Yard Scale & Scalehouse

Perry,

This design is based on drawings provided by T&R Custom Incorporated, Trailer S/N 1841355 and Minute Man product THDHLS concrete anchor.  All drawings and details are attached.  The calculated pressure(s) are based on an Ultimate Wind Speed of 144 mph and a Nominal Wind Speed of 112 mph per IBC 2012 para 1609.3.1.

To support the pressure(s) from 144 mph winds, the trailer shall have a minimum of 10 each tie down straps with achors on each side of the trailer and 2 each tie down straps on each end of the trailer.  The tie down straps shall be anchored to an 8-inch-thick reinforced concrete slab under the trailer using the THDHLS Double Head Tension Device and a 5/8” wedge anchor at each tie down location.  The concrete slab shall have a minimum strength of 3,000# within 28 days. The tie down strap(s) shall be installed at a 45-degree angle to the slab.

Sincerely,

If you have any questions please contact me.

Thank You,

David Dammon

Bill Kingsmill’s contact information is as follows:

William Kingsmill (kingsmillprop@bellsouth.net)

Cell: (504) 528 3838

1200 Business Highway 190, Suite 13

Covington, LA 70433

Lawrence Wm. Haik, Sr

Republic Commercial Properties, LLC

1539 Royal Palm Dr.

Slidell, LA 70458

(985) 960-7033