The cost for renovations to the sewer laterals at BCCY depend on the ability to use trenchers and backhoes vs hand digging.  The following trees by number would be affected by using a trencher:

#52 near the Admin and Gym outside the fence.  The Risk Assessor recommended, to remove and not to grind the stump.

#61 front of Bldg K.  The Risk Assessor recommended, Due to the tree being immature is recommended for removal before it becomes too large. Grind stump.

#62 front of Bldg K.  The Risk Assessor recommended, If  both sewer lines need to be replaced tree should be removed. If only one needs to be replaced then general recommendations.

#63 front of Bldg K.  The Risk Assessor recommended, Removal - Depends on sewer line placement.

#44 between Bldg K and the Residence #3.  The Risk Assessor recommended, remove all limbs above roof and AC unit. Provide 10' lateral buffer from edge of roof and bldg., removing limbs up to 8" in caliper.  Remove one large limb with 2 knobs from previous pruning cuts. This limb comes off of trunk and is approx 18" caliper.

#31 & #32 near School House Bldg 12.  The Risk Assessor recommended, Remove tree grind stump dispose of grinding's and backfill hole with River Silt

#27 next to the Old School House Bldg 11.  The Risk Assessor recommended, Remove entire tree. Grind stump. Remove stump grinding's backfilling hole with River silt.

#21 & #24 front of  Bldg J, #14 & #15.  The Risk Assessor recommended, This rating will depend on sewer line replacement.