BID PROPOSAL

ST. TAMMANY PARISH


DEPARTMENT OF ENGINEERING


Specifications for 


2004 District 1 Capital Road Improvements
Project Number:  
Project Number:

Parish Bid Number: ____________

Specification Date:

Section 00011

TABLE OF CONTENTS
Section 00011




Table of Contents

Section 00100




Instructions to Bidders

Section 00101




Summary of Work

Section 00315




Construction Proposal

Section 00510




Form of Agreement

Section 00610




Contractor’s Affidavit,  Louisiana

Section 00615




Contractor’s Affidavit, St. Tammany Parish

Section 00726




General Conditions

Section 01400




Testing Laboratory Services

Section 01500




Project Sign

Section 02510




Pavement Base and Subbase

Section 02513




Asphaltic Cement Concrete Pavement

Section 02520




Pavement Markings and Barricades

Section 02525




Pavement Shoulders, Aggregrate

Section 02725




Ditch Excavation and Incidental Drainage

Exhibit “A”



Requirements for Asphaltic Concrete Mixture

Table I, DPW Type I Wearing Course

Table II, Type 3 Wearing Course

Table III, Type 3 Binder Course
Section 00011
   Section 00100
Instructions to Bidders

1.
Bid security is required.  Be sure that your bid includes such security as is necessary to meet Parish requirements and is properly signed. The  proposal must be fully completed. All applicable Louisiana license numbers must be affixed.

2. The Owner is the Parish of St. Tammany.

3. The terms “he/his” and “it/its” may be used interchangeably.

4. The terms “Owner” and “St. Tammany Parish” may be used interchangeably.


SPECIAL INSTRUCTIONS TO BIDDERS
1.
The successful Bidder shall be responsible to provide additional warranty on all roadways which are presently still under warranty or contract with the Parish as of this date.  The successful Bidder is hereby advised that should any damage be caused to these roadways by construction activities, the cost to repair same to its existing condition shall be its full responsibility.  The Owner urges the successful Bidder to use alternative routes during construction.

2.
The Owner reserves the right to reject and remove any bid on specific roadways or portions thereof due to budgetary restrictions and/or constraints.

3.
All driveway and intersecting roadway ramps must be placed and installed simultaneously with the laying/installing of all asphalt.

4. 
The successful Bidder understands the limited contract time in the contract is
 Ninety (90) calendar days, and shall submit on a weekly basis any request for an extension of time.  Said request will reflect the days requested and the reason for same. Extensions thereof may be granted as per law. No extension request is guaranteed or absolute. The Owner shall have exclusive and unilateral discretion to accept, deny, modify or reject any such request.



5. Bidder specifically understands that acknowledgment of the General Conditions is required. Bidder specifically understands that signature of receipt of the General Conditions is mandated. Failure to acknowledge receipt or failure to affix the appropriate signature to the General Conditions shall be cause to deny or reject any bid.

6. 
The contractor shall select a testing laboratory.  Contractor shall submit in writing, the name of the selected testing lab for the Department of Engineering’s approval. It is the contractor’s responsibility to provide the testing laboratory  with the testing requirements and schedule for the project as outlined in the project specifications.  A copy of the test results shall be submitted to St. Tammany Parish Engineering Department  along  with the contractor’s application for payment.  
Section 00100
Section 00101
Summary of Work

1.00

Work to Include:     
Soil cement stabilization, asphalt cement surfacing, and roadside drainage.

2.00

Location of Work:
    

The site of the proposed work is located in District 1,   

St. Tammany Parish, State of Louisiana.                                             

3.00

Documents:
    

Bid documents dated September 23, 2002 and entitled: 

2004 District 1 Capital Road Improvements

Project Number:  

Fitzsimmons Road;   R01C001;   Work Order Number:  28006

4.00

OTHER REQUIREMENTS
    

When not otherwise specified herein, all work and materials shall conform to the requirements of the Louisiana Department of Transportation and Development hereafter called LDOTD (specifically 1992 Edition of Louisiana Standard Specifications for Roads and Bridges).

The Contractor shall accomplish and maintain one of the following prior to paving:

1.
On a prepared, graded and compacted non-paved road bed:  Apply a "prime coat" to said surface.

2.
On an existing paved surface:  Apply a "tack coat" to said surface.

Reference:  "Louisiana Standard Specifications for Roads and Bridges", 1992 Edition, Sections 302, 501, and 504.

Section 00101
Section 00315
Construction Proposal
CONSTRUCTION PROPOSAL FOR

Project Bid Number:

2004 District 1 Capital Road Improvements
Project Number:  
ST. Tammany Parish, Louisiana


_____________________________

Contractor

_____________________________

Louisiana License Number

_____________________________

Date

THIS BID IS SUBMITTED TO:

ST. Tammany Parish, Louisiana

Department of Administration

Purchasing Division

P.O. Box 628

21490 Koop Drive

Mandeville, LA  70471

Gentlemen:

Having thoroughly examined the drawings, the specifications, and the site of the proposed work, the undersigned, in compliance with your Invitation to Bid, hereby proposes to construct the work in full compliance with the Contract Documents as prepared by St. Tammany Parish Department of Engineering, and

Having fully and completely examined the site and locality where the work is to be performed, the necessary legal requirements, together with all Federal, State and Local laws, ordinances, rules and regulations and the conditions affecting cost, progress or performance of the work and its interpretation to the work that will be done by others as shown on the plans, and having made such independent investigations as the undersigned  deem necessary, and

This bid is genuine and not made in the interest of or on behalf of any undisclosed person, firm or organization and is not otherwise submitted in conformity with any agreement or rules of any group, association, organization or corporation, and we have not directly or indirectly induced or solicited any other bidder to submit a false or sham bid and we have not solicited or induced any person, firm or corporation to refrain from bidding and have not sought by collusion to obtain for ourselves any advantage over any other bidder or other owner, and

We will complete the work for the following price(s)

Section 00315-1
The undersigned agrees that the bid will remain open and in effect for thirty(30) days after the day of bid opening, and agrees, if this bid is accepted, to enter into an agreement (contract) with the Owner within thirty (30) calendar days from notification of acceptance of this bid in the form included in the Contract Documents to complete all work as specified or indicated in the Contract Documents for the Contract Price(s) and within the Contract Time indicated on this bid and in accordance with the Contract Documents.

The undersigned fully understands and agrees that time is of the essence and guarantees completion of all work included in this proposal within Ninety (90) calendar days from receipt of the notice to proceed.
Bid security attached, in sum of five  (5%) of this proposal, in the amount of                                                                                                                                 Dollars ($_________________).

The undersigned agrees to furnish a surety bond and Insurance Certificates in conformance with contract requirements to the Owner prior to contract signing.

The undersigned, if the successful bidder, agrees to execute a contract in the form prescribed and to execute in suitable notarial form attesting that this work was not secured through payment to anyone.

The undersigned, if the successful bidder, agrees to the terms and amount of liquidated damages of $100.00 for each and every day in excess of the time for completion of the work.

The undersigned, if the successful bidder, fully understands and agrees that a reasonable attorney fee and all related costs may be assessed against the Contractor in the event of default or termination of services.

The undersigned agrees to provide a copy of any and all resolutions (if submitted by a corporation or partnership) authorizing the undersigned to enter and sign this proposal.

The undersigned understands that failure to do so may disqualify the undersigned as a result of such non-production.

Receipt is acknowledged of Addenda No.(s)                           , and information contained therein has been utilized in the preparation of the bid.

The undersigned understands and agrees that the Parish may, at its exclusive option and discretion, nullify, rescind, or otherwise cancel this agreement if the Parish determines in its exclusive and unilateral discretion that the undersigned is not responsible and/or responsive to the needs of this Parish and/or the terms of this agreement.

I hereby certify that this firm is licensed in accordance with Louisiana Law, specifically Act 233 of 1956, as amended, and companion laws generally called 'Contractor Licensing Law' and furthermore that this firm is licensed in the proper classification to perform the work proposed and that the firm's license number is                                                              .

Section 00315-2
The undersigned specifically understands that acknowledgment of the General Conditions is required. Bidder specifically understands that signature of receipt of the General Conditions is mandated. The undersigned duly acknowledges receipt of the General Conditions. The undersigned affixes its signature attesting that the General Conditions have been received, reviewed, understood and agreed to in all respects.

The undersigned warrants that he/she has the requisite authority to sign this documentation. 

Respectfully submitted,
                                                                                    



           Firm Name

                                                                                    




By
                                                                                    




Title
                                                                                    




Address


Section 00315-3
Section 00510
Form of Contract

Between Owner and Contractor

Construction Contract 



United States of America

By: Parish of St. Tammany



State of Louisiana

with :____________________________

Parish of St. Tammany

This agreement entered into this ________day of _____________________________, 2003, by: _________________________________________, hereinafter called the “CONTRACTOR”, whose business address is________________________________________, and the Parish of St. Tammany, herein represented by William Oiler, Chief Administrative Officer, hereinafter called the “OWNER”.

Witnesseth that the Contractor and the Owner, in consideration of premises and the mutual covenants; consideration and agreement herein contained, agree as follows:

Statement of Work:
The Contractor shall furnish all labor and materials and perform all of the work required to build, construct and/or complete in a thorough and workmanlike manner:

_____________________________________________________________________________.  It is recognized by the parties herein that said Construction Documents, including by way of example and not of limitation, the Plans and Specifications, the General Conditions, Supplementary General Conditions, and addenda thereto, and the drawings (if any), impose duties and obligations upon the parties herein, and said parties thereby agree that they shall be bound by said duties and obligations.  For these purposes, all of the provisions contained in the aforementioned Construction Documents are incorporated herein by reference with the same force and effect as though said Construction Documents were herein set out in full.  Signed copies of the aforementioned Construction Documents are in the possession of both the Contractor and the Owner for reference.

Time for Completion:      The work shall be commenced on a date to be specified in a written order of the Owner and shall be completed within Ninety (90)  calendar days from and after said date.

Compensation to be paid to the Contractor:  The Owner will pay and the Contractor will accept in full consideration for the performance of the contract the sum of :

Section 00510-1
PERFORMANCE AND PAYMENT BOND:
To these present personally came and intervened:

_______________________________________________________________, herein acting for _________________________________________________________, a corporation organized and existing under the laws of the State of ___________________________________, and duly authorized to transact business in the State of Louisiana, as surety, who declare that having taken cognizance of this contract and of the Construction Documents mentioned herein, he hereby in his capacity as its Attorney in Fact obligates his company, as Surety for the said Contractor, unto the said Owner, up to the sum of :                                                                                                     

_____________________________________________________________________________.

The condition of this performance and payment bond shall be that should the Contractor herein not perform the contract in accordance with the terms and conditions hereof, or should said Contractor not fully indemnify and save harmless the Owner from all costs and damages which he may suffer by said Contractor’s non-performance or should said contractor not pay all persons who have fulfilled obligation to perform labor and/or furnish materials in the prosecution of the work provided for herein, including by way of example, workmen, laborers, mechanics and furnishers of materials, machinery, equipment and fixtures, then said surety agrees and is bound to so perform the contract and make said payment(s).

Contractor and Owner specifically agree to and recognize (1) the statutory employer relationship existing between the Owner and any employees performing work under this construction contract as employees of the Contractor or employees of the “Sub-Contractor”, and (2) that the work performed by the employees of the Contractor and the employees of the “Sub-Contractor” is part of the Owner’s business, occupation or trade and is essential to the ability of the Owner to generate their products or services, all of which is in accordance with Act 325 of the 1997 Legislative Session amending La. R.S. 23:1061, and as may be amended, as per law.

Provided that any alteration which may be made in the terms of the contract or in the work to be done under it, or the giving by the Owner of any extensions of time for the performance of the contract, or any other forebearances on the part of either the Owner or the Contractor to the other shall not in any way release the Contractor or the Surety from their liability hereunder, notice to the Surety of any such alterations, extensions or other forbearance being hereby waived.

Section 00510-2
In Witness thereof, the parties hereto on the day and year first above written have executed this agreement in _______________________________________ counterparts, each of which shall, without proof or accountancy for the other counterparts, be deemed an original thereof.

Witnesses:





Contractor:

_______________________



By:________________________

_______________________



Title:_______________________                                                                                                  
        

 __________________________________

William Oiler/Chief Administrative Officer

Parish of St. Tammany

___________________________________






(Surety)

BY: _______________________________






Attorney In Fact

APPROVED FOR LEGAL FORM AND ADEQUACY.

___________________________________

Section 00510-3
Section 00610
Contractor’s Affidavit

as Required by Act 38,

1965 Louisiana State Legislature

St. Tammany Parish

Department of Administration

Purchasing Division

Post Office Box 628

Covington, Louisiana  70434

I, __________________________________, representing_______________________________

___________________________________, in ________________________________________ as its _______________________________________ certify in compliance with ACT 38, 1965 of the Louisiana Legislature that a Contractor for the

and that he does hereby certify in compliance with La. R. S. 38:2224, that he has employed no person, corporation, firm, association or other organization, either directly or indirectly, to secure the contract for the above-mentioned. . . . . . persons regularly employed by him whose services in connection with the construction of said public project or in securing the contract for same were in the regular courses of their duties for him; and, that no part of the contract price received, or to be received, by him as paid or will be paid to any person, corporation, firm association or other organization for soliciting the contract, other than the payment of their normal compensation to persons regularly employed by him whose services in connection with the construction of said public project were in the regular course of their duties for him.

APPEARER FURTHER DECLARES, that he will, in all respects, comply with the public contract laws of the State of Louisiana, including Title 38 of the Louisiana Revised Statutes, and particularly Section 2224 of said Title 38 of the Louisiana Revised Statutes.

#2224.  Affidavit attesting that public contract was not secured through employment or payment of solicitor.

A.
All architects, engineers, contractors, subcontractors, or any person, corporation, firm, association, or other organization receiving value for services rendered in connection with the construction of a public building or project shall execute an affidavit attesting.

(1)
That affiant employed no person, corporation, firm, association, or other organization, either directly or indirectly, to secure contract under which he received payment, other than persons regularly employed by the affiant whose services in connection with the construction of the public building or project or in securing the public contract were in the regular course of their duties for affiant:  and

(2)
That no part of the contract price received by affiant was paid or will be paid to any person, corporation, firm, association, or other organization for soliciting the contract, other than the payment of their normal compensation to persons regularly employed by the affiant whose services in connection with the construction of the public building or project were in the regular course of their duties for affiant.

B.
No public contract shall be granted to any person, corporation, firm, association, or other organization refusing to execute the affidavit required by Subsection A above.

Contractor has not directly or indirectly engaged in any collusion with any other bidder in the preparation or furnishing of this bid proposal.

Section 00610-1
APPEARER FURTHER DECLARES that it understands that the terms “he/him” shall be used interchangeably with “it/its.”

APPEARER FURTHER DECLARES, that it will, in all respects, comply with the public contract laws of the State of Louisiana, including Title 38 of the Louisiana Revised Statues, and particularly Section 2224 of said Title 38 of the Louisiana Revised Statutes.

_____________________________________

                                                                       BY:

Sworn to and subscribed before me this ___________day of ________________________,2003.

 _____________________________________

                     NOTARY PUBLIC

Section 00610-2
Section 00615
Contractor’s Affidavit, St. Tammany Parish
STATE OF LOUISIANA

PARISH OF ST. TAMMANY

A F F I D A V I T

I hereby certify that I am (the)______________________________________ and duly authorized representative of the firm of __________________________________ which address is ________________________________________________, and that neither I nor the above firm I herein represent has:

(a)
employed of retained for a commission, percentage, brokerage contingent fee, or other consideration any firm or person (other than a bona fide employee working solely for me or the above consultant) to solicit or secure this public contract,

(b)
agree, as an express or implied condition for obtaining this public contract, to employ or retain the services of any firm or person in connection with carrying out the contract, or

(c)
paid, or agreed to pay, to any firm, organization or person (other than a bona fide employee working solely for me or the above consultant) any fee, contribution donation, or consideration of any kind for, or in connection with, procuring or carrying out this public contract;

I acknowledge that this affidavit is to be furnished to the PARISH OF ST. TAMMANY in connection with this contract in compliance with the provisions of Louisiana Revised Statutes 38:2224 and is subject to applicable State and Federal Laws, both criminal and civil.

________________________________

______________________________

                       Date




                      Signature

SWORN TO AND SUBSCRIBED BEFORE ME, at                                _________________________ this __________________day of ___________________________,2003.

BY:                                                                            

                                                                                                   NOTARY PUBLIC
Section 00615
Section 01400


TESTING LABORATORY SERVICES
1.0 
General

A.
An independent testing laboratory shall be retained by the contractor and approved by St. Tammany Parish Department of Engineering for the purpose of sampling and testing of materials and/or performing inspections.

B.
Cooperation of the Contractor

The Contractor shall cooperate with the laboratory and shall:

1.
Make available without cost samples of all materials to be tested in accordance with applicable standard specifications.

2.
Furnish such nominal labor and sheltered working space as is necessary for testing laboratory to obtain samples at the project site.

3.
Advise the laboratory of the identity of material sources and instruct the suppliers to allow tests or inspections by the laboratory.

4.
Notify the laboratory sufficiently in advance of operations to allow for completion of initial tests and assignment of inspection personnel.

5.
Notify the laboratory sufficiently in advance of cancellation of required testing operations.  The Contractor shall be responsible to the Parish for charges due to failure to notify it requirements for testing are cancelled.

2.0
 Testing Methods

A.
Test and inspections shall be conducted in accordance with the latest applicable ASTM, ACI, AASHTO, or LDOTD requirements, or the requirements of other recognized authorities.

B.
Rejection of Materials

1.
The laboratory shall notify the Contractor of any materials which are not in full conformance with the specifications.  The Project Engineer shall be informed of such notifications.

C.
Test Reports

The laboratory shall promptly submit written reports of each test and inspection made to the Parish, Project Engineer, Contractor and to such other parties the Engineer may specify.  Payment applications will not be processed until Department of Engineering receives test data for the payable items of the payment application.  If test reports are not received prior to final acceptance then there will be an automatic 50% reduction in pay for the item(s) in question.

D.
Shoulder Material

If test data for the shoulder material does not meet the specifications then there shall be a reduction in pay for that item by 10%.

Section 01400-1
3.0
 Portland Cement Concrete

3.1
 Tests and Inspections
A.
Mix Design

1.
The laboratory shall be responsible for the design of each specified mix.

2.
The design shall be based on a statistical analysis provided by a qualified testing laboratory or by trial batches which will be performed by the designated laboratory.  Both methods shall be in accordance with and  limited to the requirements set forth by the American Concrete Institute (ACI) 318-86, Chapter 4.

3.
Of trial batches are required, sufficient time must be given to the laboratory to perform all tests that are required.

4.
Regardless of what method is employed, the following tests are required prior to use on a given project:

ASTM C-136 - 
Sieve Analysis for Fine & Coarse Aggregates

ASTM C-127 - 
Test Method for Specific Gravity & Absorption of Coarse Aggregate.

5.
Certificates of Delivery which include test results shall be submitted by the concrete producer for cement, fly ash and additives to the testing laboratory for review.  These certificates must certify that the material conforms to the specifications.

B.
Batch Plant

1.
Prior to initial start-up of pavement operations for the project the testing laboratory shall visit the concrete suppliers batch plant to determine if the operation substantially conforms to the requirements of ASTM C94.  Any substandard conditions observed shall be immediately reported to the Engineer and the Contractor.

2.
Plant inspector is not required to be present at the plant during production.  However, the Engineer may require an inspector to be provided periodically at his discretion.

3.
The plant must be approved prior to beginning production for a given project.  This approval will be given by the Engineer and/or his representative.   Conditions for approval shall include but not be limited to the following:

i.
ASTM C-94 (latest revision) Standard Specifications for Ready Mixed Concrete.

ii.
Whenever fly ash is used, the plant shall have a separate silo or bin to store the fly ash.

iii.
Prior to and periodically during construction the laboratory shall sample and test aggregates proposed for use in concrete to determine their compliance with these specifications.

C.
Roadway Inspection

1.
The laboratory shall provide a qualified technician at the job site during all times concrete is being placed.  This technician shall be responsible to perform the required field tests and promptly notify the Contractor of any substandard materials or workmanship being incorporated in the work.

Section 01400-2
2.
Samples of fresh concrete shall be gathered in accordance with ASTM C-172 (latest revision).  

3.
Test for slump:  ASTM C-143 (latest revision);   Test for air content : ASTM C-173 or C-231 (latest revisions) if applicable, must be performed whenever test cylinders are taken, and more frequently when deemed necessary.

4.
The concrete temperature shall be measured and recorded when each set of cylinders is molded.

5.
Test cylinders shall be cast in accordance with ASTM C-31 (latest revision) as follows:

i.
Pavements:
One set consisting of four (4)  cylinders shall be cast for each 100 cubic yards or fraction thereof.  A minimum of four (4) cylinders shall be cast per visit.

ii.
Curbs:

One set of four (4) cylinders shall be cast in the A.M. and one set shall be cast in the P.M.  A minimum of four (4) cylinders shall be cast per visit.

iii.
Curb & Gutters:
One set of four (4) cylinders shall be cast for each 50 cubic yards or any fraction thereof.  A minimum of four (4) cylinders shall be cast per visit.

6.
Test cylinders are to be tested in accordance with ASTM C-39 (latest revision) as follows:

2 @ 7 days of age 

2 @ 28 days of age

7.
During production, if any truck of ready mixed concrete fails to conform to specifications, the Laboratory will immediately notify the Contractor and the Director of Streets and this material shall not be incorporated in the work.

8.
Re-tempering of concrete is not permitted.

D.
Cores for Thickness Verification

1.
Cores shall be taken by the Laboratory no later than 30 days after the conclusion of paving operations.

2.
A minimum of one (1) core shall be obtained from each three hundred (300) linear feet of each pavement lane.

3.
Final locations of the cores shall be reported in a manner than is acceptable to the Engineer.

4.0
Asphaltic Concrete Mixtures

A.
Tests and Inspections:

1.
The Contractor will be required to design the mixtures for optimum asphalt content and comply with all requirements of LDOTD designation TR 303, Method A, however, Method B may be used when approved by the Parish's Engineer.  Mixes which have been previously approved and are current within six months may be submitted for approval subject to the same criteria as required by TR 303, Method A.

Section 01400-3
2.
The Job Mix Formula shall be submitted for approval to the designated Parish Testing Laboratory and the Engineer, supported by appropriate design data.  No mixture shall be produced for the job until the Contractor's Job Mix Formula has been approved by the Engineer.  Approval or rejection of Job Mix Formula shall be accomplished within seven (7) calendar days of submittal.

3.
The Job Mix Formula will allow full range of gradation mix tolerances within the mix type specification limits.

4.
The plant shall be so operated as to produce on a continuing basis a mixture conforming to the approved Job Mix Formula, the requirements in Table 1 of LDOTD Section 501 and other specifications requirements.  When this is not the case, the Contractor shall begin making satisfactory adjustments or cease operations.  The Engineer may permit the Contractor to submit a new Asphaltic Concrete Job Mix for approval.

B.
Preliminary Tests

Prior to starting plant operations, the following materials will be sampled and/or tested by the Parish's designated Testing Laboratory:

1.
Obtain LDOTD  Certificates of Delivery covering asphalt cement in working tank.  (Also to be obtained on a daily basis during production.)

2.
Obtain the refinery test report covering chemical and physical properties of the asphalt cement.  

3.
Verify that the Anti-Stripping agent is included in the LDOTD Qualified Products List.

4.
Other admixtures - LDOTD Qualified Products List.

5.
Aggregates - Verify that the source of aggregates to be used are included in the LDOTD Qualified Products List.

6.
Sample the Contractor's fine sand.  Test in accordance with AASHTO. T-90 to determine if PI is zero (0).  Visually inspect Contractor's stockpile to verify absence of clay balls and excessive clay coating.

C.
Validation of the Job Mix Formula

1.
For acceptance of the Job Mix Formula, the Contractor must include the following:

a.
Design Curves (LDOTD TR 303)

b.
Apparent Specific Gravity for each material employed and its source (LDOTD TR 301)

c.
Proposed Bin Proportions

d.
Proposed A.C. Content (LDOTD TR 303)

e.
A single recommended gradation for each specified sieve size.

f.
% Crushed of Coarse Aggregate (if applicable).

2.
The Job Mix Formula shall be validated during the first day's production or when enough mixture has been produced to have four (4) Marshall Stability Tests and two (2) Extraction Tests.  In order for a Job Mix Formula to be validated, the test results must meet the requirements set forth by these specifications using design criteria for each specified mix.

Section 01400-4
D.
Tests Conducted at the Asphalt Plant

Tests to be conducted on the hot mix material at the asphalt plant will be conducted by the Parish's Designated Testing Laboratory's technician.  All tests will be conducted in accordance with LDOTD specifications.  Test results shall conform to the properties in Table 1 of Section 501.  A sampling plan will be utilized such that samples are taken and tested as follows:

1.
0 to 200 tons - Full sample is taken upon which the following tests are to be conducted:

Temperature - Read to the nearest 5 degrees.

Extraction

Extracted Gradation


TR-309

% Extracted Asphalt Cement

TR-308

% Crushed



TR-306

Marshall Stability & Flow


TR-305

(Two briquettes are made.)

Unit Weight, Voids and %  VFA

TR-304

Record Anti-Strip content from meters and scales.

2.
201 tons to 400 tons - Smaller sample is taken.

Temperature - Read to the nearest 5 degrees.

Marshall Stability & Flow


TR-305

(One briquette)

3.
401 tons to 600 tons - Full sample taken.  Conduct tests as described for 0 to 200 tons.

4.
In excess of 600 tons - Smaller sample taken.  Conduct tests as described for 201 to 400 tons.

E.
Mix temperatures at the time of molding the briquettes shall be within 15 degrees Fahrenheit of the mix temperature at the time of sampling.

1.
In addition to the above, the temperature of the mix is to be taken at least once per hour, when the mix has been loaded into haul trucks and is ready to leave the plant.

2.
If the test results during the production of a lot are outside the limits given in Table 1 of Section 501 of the Parish's Testing Laboratory, a representative shall immediately notify the Contractor's qualified technician.

3.
When an individual  test or the average of tests representing the lot is outside acceptance limits shown in Table 1 of LDOTD Section 501, an adjustment in unit price for the lot will be made in accordance with Table 2 of LDOTD Section 501.

Section 01400-5
F.
Pavement Samples

1.
The density requirements will be for the average of all cores for each lot per project, determined in accordance with LDOTD  TR-304.  Payment will be made in accordance with Table 2 of LDOTD Section 501.

2.
Samples that result in less than 1 1/4" thickness will not be tested for density.

G.
Roadway Inspection

Inspection of Roadway Asphaltic Concrete Work at the job site shall be performed by a qualified roadway technician furnished by the Testing Laboratory.  his duties will include, but not be limited to, the following:

1.
Record locations, tonnage, type of mix, lot number, and other pertinent data in his daily report.

2.
Temperatures will be checked and recorded in accordance with these specifications during the production of each lot.

3.
Observe the general operations of the Contractor to assure compliance with all requirements.

4.
Observe and record the tack and prime coat operations.  (Square yards covered, number of gallons used).

5.
Determine the location of cores to be taken by the Contractor for thickness and density.

6.
Witness the drilling or cutting of the roadway samples and deliver same to laboratory for further testing.

7.
Witness the surface finish testing performed by the Contractor and report the findings of such testing.

8.
Inspect all mix hauled to the roadway for any obvious deficiencies which may include uncoated aggregate, segregated mixtures, mixtures with lumps, mix not of the proper temperature, excessive moisture, color and general appearance of the mixtures.  Any loads found deficient shall be brought to the attention of the Contractor and rejected.  Also no trucks will be accepted without a haul ticket.  

9.
Observe weather conditions and advise the Contractor of weather limitations when they apply.

10.
Check actual yield of mixture during production to assure proper mat thickness.  The Contractor should be advised of improper thickness and corrections shall be made.

5.0
Base Rehabilitation

5.1
Materials
A.
The testing laboratory shall test representative samples of materials to confirm compliance with specification.

B.
Representative samples of pulverized base shall be tested for gradation.

C.
Representative areas of compacted base shall be tested for compaction.

Section 01400-6
6.0
 Precast Concrete Bridge

A.
Concrete

1.
The testing laboratory shall prepare a minimum of four (4) 6" x 12" sample cylinders for each fifty (50) cubic yards of concrete cast, with a minimum of four (4) samples cast per day.  If only one (1) element should be cast in one day's work, four (4) samples shall be prepared.

B.
Piles

1.
All treated piles shall be inspected by the testing laboratory for retention of preservative at the treatment facility.  Contractor shall notify testing laboratory forty-eight (48) hours in advance of this work.

2.
The testing laboratory shall verify all pile dimensions prior to driving of piles.

3.
The testing laboratory shall log the driving of all piles and submit the driving logs to the Engineer and Contractor within twenty-four (24) hours after driving.  Contractor shall notify Engineer and testing laboratory twenty-four (24) hours in advance of pile driving.

C.
Precast Concrete

1.
The Contractor shall notify both the Engineer and the testing laboratory twenty-four (24) hours prior to castings of any concrete elements.

2.
Reports of all laboratory and field inspections shall be furnished to the Engineer within forty-eight (48) hours after the inspection or test.  A copy of each report shall also be furnished to the Contractor and the Owner.  Reports shall otherwise be kept confidential.

Section 01400-7
Section 01500


PROJECT SIGNS
1.6_ General

A.
Work Included:  Constructing and placing job sign at the beginning of the project.

2.0
Materials

A.
The job sign shall be 3/8" exterior plywood (4' x 8').

B.
The lettering shall be black on a white background.

3.0
Execution

A.
The sign is to be supported such that the bottom of the sign is a minimum of 5' above the existing ground elevation.

B.
Exact placement location of the job sign is to be coordinated between the Parish, Engineer, and Contractor.

C.
The sign is to be maintained throughout the period of construction.

D.
Sign layout described below.

4.0
 Measurement and Payment

A.
Job sign and support system shall be a no direct pay item.

Section 01500
Section 02510


PAVEMENT BASE & SUBBASE, RECONSTRUCTED
1.1_ General

1.1
Description

A.
Work Included:

1.
Removing and replacing existing base, subbase and subgrade where required.

2.
Breaking and pulverizing existing asphalt pavement and mixing with existing base to form new base.

3.
Adding gravel and sand and mixing where required.

4.
Compacting and shaping.

5.
Stabilizing with cement or lime where noted.

1.2
Definitions
A.
Base:

That engineered select layer of material which lies above the subbase, or subgrade as the case might be, upon which the pavement is constructed.

B.
Subbase:
That engineered select layer of material between the subgrade and base.

C.
Subgrade:
The existing or insitu soils upon which the subbase or base is constructed.

1.3
References
A.
Those parts of the referenced specifications which are applicable hereto shall be considered as if written herein in full.

1.
LDOTD:  Louisiana Standard Specifications for Roads and Bridges, 1992 Edition as amended, and the "Highway Specifications Workbook", Latest edition.

2.
ASTM:  American Society of Testing Materials.

3.
ACI:  American Concrete Institute

4.
AASHTO:  American Association of State Highway and Transportation Officials "Standard Specifications for Transportation, Materials and Methods of Sampling and Testing".

1.4
Quality Assurance
A.
Use adequate numbers of skilled workmen who are thoroughly trained and experienced in the necessary crafts, and who are completely familiar with the specified requirements and the methods needed for proper performance of the work of this section.  

1.
During all of the time that this work is in progress the Contractor must have present at the work site a foreman or superintendent who is not an equipment operator.

Section 02510-1
B.
Use only equipment which is specifically manufactured and properly sized for the work to be performed.

1.
Vibrating rollers cannot be used except with the written approval of the Engineer.

2.
The gradation of the pulverized base is essential in obtaining proper compaction.  The proper gradation can generally only be obtained by the use of certain types of equipment.  A pulverizer or rotary mixer can produce the proper gradation.

2.0
Materials

2.1
General Requirements
A.
Sand shall be a natural deposit or pumped material with ninety-five percent (95%) by weight falling between the No. 16 and the No. 200 screen sieve.  It shall have a maximum liquid limit of twenty-five (25) and a maximum plasticity index of six (6) and be free of trash, roots and weeds.

B.
Sand-clay-gravel shall conform to the requirements of LDOTD 1003.03a.

C.
Shell shall conform to the requirements of LDOTD 1003.03b.

D.
Crushed stone and crushed concrete shall conform to the requirements of ASTM 2940.

E.
Stone shall conform to the requirements of LDOTD 1003.03d.

F.
Cement shall conform with the requirements of ASTM C150 Type 1.

G.
Fly ash shall conform to the requirements of ASTM C618 Class C.

H.
Lime shall meet the requirements of LDOTD 1018.03.

2.2
In-Place Material Requirements
A.
Reconstructed base shall be a homogeneous mixture of pulverized asphalt and base material and, when specified or noted, cementitious material, compacted to a minimum of 95% of the maximum theoretical density as determined by ASTM, D1557.

B.
Sand used as a base or subbase material shall be compacted to a minimum of 96% of ASTM D698.

C.
Except as included herein or as noted on the plans all work shall conform with the requirements of Part III of LDOTD  "Standard Specifications for Roads and Bridges" and except as specified herein, density requirements shall be those specified in 301.15 of LDOTD.

Section 02510-2
3.0
Execution

3.1
Test Section
A.
Base Mixture

1.
Prior to beginning work the Contractor shall make one or, if directed by the Engineer, two test sections of pulverized base using the equipment and procedure that he proposes to use for the work.  The materials testing laboratory shall obtain samples of the homogeneously mixed material and run gradation and Atterburg limit tests.  Work on the base will be delayed until the Engineer has a chance to study the results of the laboratory tests but the delay shall not exceed five (5) days.

2.
The procedure used by the Contractor to pulverize and mix the base shall produce a mixture conforming to the following requirements:

Screen Size



Percent Passing
2" x 2"



100% minimum

1/2"




90% maximum

#4




45% minimum

#200




25% maximum

3.
The approved percentage of material passing the screen sizes may be varied by the Engineer.

4.
The Engineer, at his discretion, may require the Contractor to add sand and gravel to the base to be thoroughly blended into the mixture.

3.2
Scarifying, Pulverizing and Mixing

A.
Where noted on the plans and where directed by the Engineer the existing asphalt surfacing shall be broken, pulverized, mixed with the existing base, shaped and compacted.

B.
Unless otherwise approved by the Owner, the existing asphalt surfacing shall first be scarified and broken into pieces, then pulverized and thoroughly mixed.

1.
Gravel or sand will be added to the pulverized base, if so directed by the Engineer, and the mixing continued until it is homogeneous.

2.
Cement or lime, as specified, will be added at this time to the pulverized base and then thoroughly mixed with the proper amount of water.

C.
Mixing shall be performed by a preparizer traveling pugmill or a single or multiple shaft rotary mixer capable of breaking and thoroughly mixing the asphalt, base and any added material into a uniform homogenous mixture of the gradation required and thickness shown on the plans or directed by the Engineer and when compacted of not less than six (6") inches or more than eight inches (8") in thickness.

1.
Exercise care not to pick up any deleterious material from under the base.

2.
The material or materials to be used to modify the existing base will be uniformly spread over the surface to be improved in the amounts specified or directed and thoroughly blended into the mixture.

Section 02510-3
3.
The base shall be combined with lime or cement and water only by a traveling plant.  The spreading of bulk lime or cement onto the base is not permitted except with written permission of the Engineer.  This permission will not be given when a dusting problem can exist or when the roadway must be open to traffic while this operation  is in progress.

4.
Pulverization and mixing and cement and/or lime mixing shall be separate operations.

E.
Only six hundred (600) feet of base will be open at a time without the written permission of the Engineer.  Mixing, compaction, grading and curing shall take place before the Contractor opens up another section of base.

F.
When not in conflict with this section the requirements of LDOTD will govern.

G.
When the base is modified with cement or lime the Contractor shall keep the roadbase closed to construction traffic for 72 hours.  Local traffic will be allowed on the base during this period.

3.3.
Shaping and Compacting
A.
Potable water, in the amount required to obtain maximum compaction, shall be added and mixed into the base.

B.
Mixing shall continue until uniformity is reached.

C.
Compaction, shall follow directly behind the mixing without interruption and shall continue until the base is uniformly compacted to a minimum of ninety-five percent (95%) of theoretical maximum density as determined by ASTM D1557, modified proctor.

D.
Required densities must be obtained within three (3) hours of mixing cement into the base.  If the Contractor demonstrates his ability to obtain the required density within the three hour period then the Engineer will consider enlarging the six hundred (600) foot limit.  Conversely, if the Contractor fails to obtain the required densities within three hours within the entire six hundred (600) foot limit then the Contractor shall, at no expense to the Owner, reduce the six hundred (600) foot limit to three hundred (300) feet.

E.
The finished base shall have a smooth, uniform, closely knit surface, free from ridges, depressions or loose material.  The finished base shall conform to the thickness, lines, grades and side slopes shown on the plans.

F.
The finished base shall be protected by the application of an asphaltic curing membrane conforming to the requirements of LDOTD Section 506.

G.
Prior to the application of the asphaltic cement concrete, the surface of the base where damaged shall be replaced.

H.
The Contractor can elect to keep the base moist by watering for the first 72 hours instead of applying the asphaltic curing membrane.  The design and weight of the watering truck must be approved by the Engineer.

I.
If more than fourteen (14) days pass between completion of the base and the application of the asphalt, an additional application of asphalt curing membrane shall be applied prior to surfacing and in accordance with LDOTD Section 506.

J.
Contractor shall be careful in the section of his equipment to use the lightest available in order to avoid pumping or damage to the base.

Section 02510-4
4.0
Subbase Repairs
A.
Wherever noted on the plans or directed by the Engineer, the Contractor shall excavate and remove roadway base and subbase in order to establish a firm surface upon which to build the roadway.

1.
Excavate to the depths shown on the plans, directed by the Engineer or, if not so noted, to a depth of 24" below the top of pavement.

2.
Replace excavated deleterious material with sand conforming to 2.1A or such other material as directed by the Engineer.

B.
Place sand fill in loose layers of 12" maximum thickness and compact to 97% of ASTM  D698.

1.
Do not replace or construct base until subgrade complies with density requirement.

5.0
Measurement and Payment

A.
Breaking, pulverizing and mixing into the existing surfacing and base, shaping, compacting, curing and maintaining until surfacing is in place shall be measured by the square yard and paid for at the contract unit price for base complete in place.

B.
Gravel, sand or shell required to be added to the base shall be measured by the cubic yard net section.  Payment for this material shall be by the cubic yard net section at the contract price or if no contract price exists then payment shall be at a mutually agreed upon price.  If no price can be mutually agreed upon then the Owner may deliver and spread upon the base the required aggregate.  There shall be no direct payment for mixing.

C.
Cement, lime, asphalt, etc. added, mixed and compacted into the base shall be measured as indicated on the proposal and paid for at the contract unit price complete in place.

D.
All other costs associated with the base work including curing, water etc. shall be non-direct pay items.

E.
Subgrade replacement shall be measured by the square yard.  Payment shall be at the contract square yard price for removal and disposal of material to the depth shown on the plan (24" if not shown) and the replacement with acceptable material compacted to required density.

Section 02510-5
Section 02513


ASPHALTIC CEMENT CONCRETE PAVEMENT

1.0
General

1.1
Description
A.
Work included:

1.
Hauling, spreading and compacting the asphaltic cement concrete to the lines, grades, sections and thicknesses shown on the plans.

B.
Work not included:

1.
Base repair and drainage.

1.2
References
A.
Those parts of the referenced specifications which are applicable hereto shall be considered as if written herein in full.

1.
LDOTD:  Louisiana Standard Specifications for Roads and Bridges, 1992 Edition as amended, and the "Highway Specifications Workbook", Latest edition (Section 501 and any related sections).

2.
ASTM:  American Society of Testing Materials.

3.
ACI.:  American Concrete Institute

4.
AASHTO:  American Association of State Highway and Transportation Officials "Standard Specifications for Transportation, Materials and Methods of Sampling and Testing".


2.0
Materials

A.
Asphaltic cement concrete shall conform with all of the requirements of LDOTD for the various types shown on the plans.  95% density shall be required in lieu of 96% as required by Silver Book & Table and in the Silver Book these be adjusted accordingly.

B.
Recycled asphalt is disallowed for use in asphaltic mix designs for any materials being placed on parish projects per Resolution Police Jury Series No. 89-4073.

3.0
Execution

A.
All work shall be in conformance with the requirements of LDOTD.

B.
All intersecting roads and streets to be constructed a minimum of 20' from the edge of roadway being overlayed, in accordance with the typical details.

C.
Minimum turning radius of 35' (where existing conditions allow) at intersecting roads or otherwise specified by the Engineer or a representative of the Engineer.

D.
All driveway aprons shall be a minimum of one (1) foot behind the back side of the culvert whether installed under this contract or not, and shall cover all fill material added for installation of new culverts.  If no culvert is present the minimum apron shall be three (3) feet.  Apron shall be placed at same time as surface course is placed and shall be sufficient to transition existing driveways to the edge of the road being overlayed.  Minimum driveway radius is twelve (12) feet.  (Per Resolution Police Jury Series No. 93-6042 adopted January 21, 1993).

Section 02513-1
4.0
Measurement and Payment

A.
Asphalt shall be measured by the square yard and paid for at the contract unit price per square yard complete in place minimum any deduction for deficiencies.

B.
No direct payment shall be made for 20' paving and construction of intersecting roads and driveway transitions in accordance with typical details.

C.
If the final product reveals areas of segregation, multiple clay lumps, ravels, or any type of rough areas/deficiencies then the area(s) in question shall be removed and replaced at the contractors expense.

5.0
Asphaltic Concrete Mixture

A.
By Resolution 89-4181 - Table 1 of "Section 501-Asphaltic Concrete Mixtures" is replaced by Exhibit A.

Section 02513-2
Section 02520

PAVEMENT MARKINGS AND BARRICADES
1.0  
General

1.1
Traffic Maintenance
A.
Provisions, approved by the Engineer, for maintaining traffic throughout the length of the project and life of the contract must be made by the Contractor.

B.
The Contractor shall keep the portion of the project being used by public traffic, whether through or local traffic, in such condition that traffic (including mail delivery) will be adequately accommodated.  He shall furnish, erect and maintain barricades, warning signs and delineators, and shall provide flagmen and pilot cars in accordance with the plans and the MUTCD.  He shall also provide and maintain in a safe condition all required temporary approaches or crossings, intersections with roads, streets, business, parking lots, residences, and garages.

C.
The Contractor shall place no more than one-half (1/2) of each days work in one lane and the remainder in the adjacent lane.

D.
At the end of each days operation, temporary pavement marking shall be placed and proper signs and barricades displayed.  During the period that all lanes are open to traffic, the Contractor shall neither store material nor park equipment on roadway shoulders.

E.
The Contractor shall be responsible for supplying, maintaining and moving, when necessary, adequate barricades, warning signs and lights to safely inform the traveling public as to the location of the work at all times.

F.
No direct payment shall be made for providing for an maintaining traffic as required herein, except as provided in the proposal.

1.2
Signs, Barricades, Pavement Markings and Markers
A.
Both temporary and permanent shall conform to latest LDOTD. specifications and the MTUCD.

B.
Pavement shall be permanently marked as shown on the plan:

1.
Thermoplastic pavement markings shall be reflectorized and conform with 1992 LDOTD. Standard Specifications.

2.
Location, size and color as shown on the plan.

3.
Painted traffic striping in accordance with 1992 LDOTD. Standard Specifications.

Section 02520
Section 02525

PAVEMENT SHOULDERS, AGGREGATE

1.0
General
1.1
Description
A.
Work included:   Removing surface vegetation, casting, spreading, shaping and compacting aggregate shoulder material.

B.
Work not included:

1.
Relocating ditches, filling existing ditches.

2.0 
Materials
A.
Shoulder material shall be one of the types of aggregate material listed below.

1.
Crushed stone LDOTD. 1003.04

2.
Sand clay gravel LDOTD. 1003.04

3.
Crushed concrete ASTM 2940

4.
Reef shell ASTM 1003.03b modified to contain a maximum of 50% clam by weight.

3.0
Execution
A.
Remove vegetation, spread, shape and compact shoulder material from edge of road to edge of ditch embankment, unless otherwise specified.  Compaction to ninety-three percent (93%) ASTM D1557 or to a lesser degree as approved by the Owner.

B.
Compacted shoulder material to match surface elevation at edge of new pavement and to be sloped toward ditch at a slope of 1/8" per foot.

C.
The minimum width of the shoulder material shall be three (3) feet from the edge of the new asphalt unless conditions exist such that the minimum width can not be obtained.
D.
Shoulder material shall be placed within thirty (30) calendar days from the date that the wearing course of asphalt is laid.
4.0
Measurement and Payment
A.
Shoulder material shall be measured by the lump sum and payment shall be at the contract unit price for shoulder material complete in place.

B.
Contractor to estimate the quantity of material needed when preparing lump sum estimate.

Section 02525
Section 02725

DITCH EXCAVATION AND INCIDENTAL DRAINAGE
1.0
General

1.1
Description
A.
Work included:  cleaning and shaping roadside ditches, constructing new ditches, cleaning existing culverts and structures and installing new structures and culverts.

1.2
Quality Assurance
A.
Use adequate number of skilled workmen who are thoroughly trained and experienced in the necessary crafts and who are completely familiar with the specified requirements and the methods needed for proper performance of the work of this section.

B.
Use equipment adequate in type, size, capacity and numbers to accomplish the work in a timely manner.

2.0
Materials

2.1
All materials shall conform to the requirements of 1992  LDOTD.  Standard Specifications for Roads and Bridges.

3.0
Execution

3.1
Cleaning Existing Ditches
A.
Clean and shape roadside ditches where shown on the plans removing all weeds and shaping so that water flows and does not stand.  Exercise care not to over excavate.

B.
Remove debris and excavated material from job site.

C.
Clean existing culverts and structures where noted on the plans.

D.
All drainage work, including ditch construction and culvert installation, shall be done in a manner than allows the ditches to drain without retaining any water.

3.2
Constructing New Ditches
A.
Excavate new ditches accurately to the lines, grades and sections shown on the plans.

B.
When not noted otherwise remove excavated material from site, including any trash, vegetation, trees, and soil.

C.
Backfill abandoned ditches with A4 or better material in twelve inch (12") layers compacting each layer to a minimum of ninety-six percent (96%) of ASTM  D698.

3.3
New Drains, Culverts and Structures
A.
Drains and culverts shall be installed where noted on the plans or as directed by the Owner.

B.
Structures shall be constructed where noted on the plans.

C.
All materials and construction, if not shown otherwise, shall conform to LDOTD requirements.

Section 02725-1
D.
Bids accepted after July 16, 1992 on roads under the Capital Improvement Program shall strictly adhere to Section 701.08 of the Louisiana Standard Specifications for Roads and Bridges, and shall incorporate Type 2  Joints as noted in Section 701.06 of the aforesaid specifications per Resolution 92-5782 as amended.

4.0
Measurement and Payment

4.1
Cleaning and Reshaping Ditches
A.
Cleaning and reshaping ditches shall be measured horizontally by the linear foot and paid for at the contract unit price per linear foot.

4.2
New Ditches
A.
New ditches shall be measured horizontally by the linear foot and shall be paid for at the contract unit price per linear foot for new ditch complete in place.

4.3
Drains, Culverts and Structures
A.
Drain pipe shall be measured by the linear foot for the various types and sizes shown on the plans or directed installed by the Owner and paid for at the contract unit price per linear foot.  Payment per linear foot includes cost of all material, excavation, foundation material, embedment material, jointing and backfill.

B.
Structures shall be measured per each and paid for at the contract unit price per structure complete in place.

Section 02725-2
EXHIBIT  "A"


TABLE I


REQUIREMENTS FOR ASPHALTIC CONCRETE MIXTURES

A.
MIXTURE REQUIREMENTS

DPW Type I


Wearing Course
U.S. Sieve Size

1 1/2"







-----

1"







100

3/4"







100

1/2"







95 - 100

3/8"







89 Max

No. 4







60 - 75

No. 8









No. 10






32 - 55

No. 40






16 - 35

No. 80






08 - 20

No. 200






3 - 9

Extracted Asphalt %

Mix Temperature

% Crushed Ret on No. 4 (8)




85 M in

Course Aggregate (1)





A, B, C

Marshall Stab (lbs)

No. of Blows





75

Average of 4 Tests

Design (2)




1400

Minimum




1200

Individual Test (Min.)


1000

Mar. Flow (1/100")





6 - 15

% Voids






3 - 5

% VFA






70 - 80


Table I

EXHIBIT  "A"
TABLE II

REQUIREMENTS FOR ASPHALTIC CONCRETE MIXTURES

A.
MIXTURE REQUIREMENTS

Type 3


Wearing Course
U.S. Sieve Size

1 1/2"







-----

1"









3/4"







100

1/2"







90 - 100

3/8"







89 Max

No. 4









No. 8







28 - 58

No. 10








No. 40








No. 80








No. 200






2.0 - 10.0

Extracted Asphalt %

Mix Temperature

% Crushed Ret on No. 4 (8)






Course Aggregate (1)





A, B, C

Marshall Stab (lbs)

No. of Blows





75

Average of 4 Tests

Design (2)




1700

Minimum




1500

Individual Test (Min.)


1300

Mar. Flow (1/100")





6 - 15

% Voids






3 - 5

% VMA






14.0

Table II
EXHIBIT  "A"
TABLE III

REQUIREMENTS FOR ASPHALTIC CONCRETE MIXTURES

A.
MIXTURE REQUIREMENTS

Type 3


Binder Course
U.S. Sieve Size

1 1/2"







-----

1"







100

3/4"







90 -100

1/2"







89 Max

3/8"







-----

No. 4

No. 8







23 - 49

No. 10








No. 40








No. 80








No. 200






2.0 - 8.0

Extracted Asphalt %

Mix Temperature

% Crushed Ret on No. 4 (8)






Course Aggregate (1)





A, B, C

Marshall Stab (lbs)

No. of Blows





75

Average of 4 Tests

Design (2)




1700

Minimum




1500

Individual Test (Min.)


1300

Mar. Flow (1/100")





6 - 15

% Voids






3 - 5

% VMA






13.0

Table III
�Forty-five





