SLIDELL TOWERS BUILDING

RESTROOM FACILITIES SURVEY

1ST FLOOR- 
ALL NEW FIXTURES AND ACCESSORIES.

2ND FLOOR-  MENS: EXISTING WC AND URINALS CAN BE CLEANED.

LADIES: WCs (ONE WALL-HUNG) CAN BE REUSED, PIPE AND OTHER FIXTURES REPLACED.

3RD FLOOR-
MENS: WCs AND PIPES NEED REPLACING, URINALS CAN BE CLEANED & REUSED.

LADIES: WCs CAN BE REUSED, PIPE AND OTHER FIXTURES REPLACED.


4TH FLOOR- MENS: WCs, PIPE, AND URINAL CAN BE CLEANED AND REUSED, REPLACE OTHER FIXTURES.


LADIES: WCs AND PIPE ON HANDICAP WC CAN BE CLEANED & REUSED, REPLACE ALL OTHER FIXTURES.

5TH FLOOR-
MENS: BOTH WCs, PIPE AND URINALS ARE IN GOOD CONDITION, REPLACE ALL OTHER FIXTURES.


LADIES: ALL 4 WCs & PIPE CAN BE CLEANED AND REUSED, REPLACE ALL OTHER FIXTURES. 


6TH FLOOR- 
MENS: REPLACE BOTH WCs AND OTHER FIXTURES.


LADIES: REPLACE ALL WCs AND OTHER FIXTURES.  REBUILD PIPE CHASE WALLS BETWEEN NEW MENS & LADIES ROOMS.

CONTRACTOR SHALL INSPECT ALL RESTROOM EQUIPMENT AND REPLACE ANY WHICH CAN NOT BE RESTORED TO “LIKE-NEW” CONDITION.

PROVIDE NEW LAVATORIES/VANITIES, PARTITIONS, MIRRORS, SANITARY NAPKIN DISPOSAL, AND TOILET SEATS.  INSTALL OWNER PROVIDED SOAP DISPENSERS, TOILET PAPER HOLDERS, AND PAPER TOWEL HOLDERS.

INTEGRAL LAVATORY/VANITY: CULTURED MARBLE, SIZE AS SHOWN ON DRAWING A-21.

URINALS: TOTO UT447 (OR EQUAL) WITH TOTO TMU1NNC 1Gpf FLUSH VALVE (OR EQUAL)

TOILETS: TOTO CT705 (CT705L FOR HANDICAP) WITH TOTO TMT1NNC-32 1.6 Gpf FLUSH VALVE, AMERICAN STANDARD 2234 MADERA WITH SLOAN 111 ROYAL 1.6 Gpf FLUSHOMETER VALVE (OR EQUAL)

TOILET SEATS: KOHLER K-4710-T (MENS) AND K-4752 (LADIES) OR EQUAL.

PARTITIONS:  BOBRICK 1080 SERIES PHENOLIC OR EQUAL

MIRRORS: SHEET MIRROR- 7’-6” WIDE (OR LENGTH OF VANITY) X 

3’-0” HIGH, MOUNTED 3’-4” ABOVE FLOOR.

SANITARY NAPKIN DISPOSAL: BOBRICK B354 (LADIES) OR EQUAL.



1/24/2007

