ASBESTOS REMOVAL AND ABATEMENT AT:

ST. TAMMANY PARISH ADMINISTRATIVE COMPLEX

Scope of Work:
   1. Properly remove and dispose of ceiling texture material throughout building containing asbestos (chrysotile), in accordance with attached specification SECTION 13280 – Removal of Asbestos Materials.

   2. See attached report on asbestos analysis.

   3. Advise Owner / Architect of any additional asbestos material encountered.

   4. Properly encapsulate ceiling texture materials not readily accessible for removal. 

Where wall plates have been attached over the texture material, texture will be scraped up to the wall plate and then the plate will be caulked at the intersection of the deck. This same procedure will be used at other surface mounted items such as electrical boxes.

Where unistrut has been attached to the deck, the unistrut will have a layer of bridging encapsulate applied over it to seal it.

A minimum of two floors should be worked at a time, from top floor to bottom.

Include complete details in asbestos abatement plan.

   5. Ceiling areas observed with ceiling texture material (approximate square footage, verify) are as follows:

First Floor         5540

Second Floor    4700

Third Floor       5420

Fourth Floor     5320

Fifth Floor        5500

Sixth Floor       5420

Pent House       Inspect and verify

Total approximate square footage = 31,900 (for estimating purposes only).

   6. Please consider this G709 Package, Work Changes Proposal Request as the response to RFI  #27.

