ST. TAMMANY FIRE PROTECTION DISTRICT NO. 1
FIRE STATION #19
57047 ALLEN ROAD
SLIDELL LA  70461


Copyright 2010 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 101423
PANEL SIGNAGE
Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 SUMMARY

A. Section Includes:

1. Room-identification signs.

1.2 ACTION SUBMITTALS

A. Product Data: For each type of product.

B. Shop Drawings: For panel signs.

1. Include fabrication and installation details and attachments to other work.

2. Show sign mounting heights, locations of supplementary supports to be provided by others, and accessories.

3. Show message list, typestyles, graphic elements, including raised characters and Braille, and layout for each sign at least half size.

Retain two subparagraphs below if signs are electrically powered.

4. Show locations of electrical service connections.

5. Include diagrams for power, signal, and control wiring.

C. Samples: For each exposed product and for each color and texture specified.

D. Sign Schedule: Use same designations specified or indicated on Drawings or in a sign schedule.

Retain "Delegated-Design Submittal" Paragraph below if design services have been delegated to Contractor.

1.3 INFORMATIONAL SUBMITTALS

A. Sample warranty.

1.4 WARRANTY

A. Special Warranty: Manufacturer agrees to repair or replace components of signs that fail in materials or workmanship within specified warranty period.

Verify available warranties and warranty periods for units and components.

1. Warranty Period: Five years from date of Substantial Completion.

PART 2 -  PRODUCTS

See Editing Instruction No. 1 in the Evaluations for cautions about named manufacturers and products. For an explanation of options and Contractor's product selection procedures, see Section 016000 "Product Requirements."

A. Accessibility Standard: Comply with applicable provisions in the U.S. Architectural & Transportation Barriers Compliance Board's ADA-ABA Accessibility Guidelines for Buildings and Facilities and ICC A117.1 for signs.

2.2 SIGNS

Copy "Manufacturers," "Basis-of-Design Product," "Panel Sign," and "Room-Identification Sign" paragraphs below and re-edit for each product.

Insert number to complete drawing designation for each product required. Use these designations on Drawings to identify each product.

Retain "Manufacturers" Paragraph and list of manufacturers below to require products from manufacturers listed or a comparable product from other manufacturers.

A. Manufacturers: Subject to compliance with requirements, provide products by one of the following:

Retain "Basis-of-Design Product" Paragraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed. Retain "Basis-of-Design Product" subparagraphs in remainder of this article if retaining paragraph below.

B. Basis-of-Design Product: Subject to compliance with requirements, provide product indicated or comparable product by one of the following:

Manufacturers listed below use one or more of graphic processes specified. Revise list to suit Project.

1. Ace Sign Systems, Inc.

2. Advance Corporation; Braille-Tac Division.

3. Allen Industries, Inc.

4. Allen Markings International.

5. APCO Graphics, Inc.

6. ASE, Inc.

7. ASI Sign Systems, Inc.

8. Best Sign Systems Inc.

9. Bunting Graphics, Inc.

10. Clarke Systems.

11. Diskey Sign Company.

12. Fossil Industries, Inc.

13. InPro Corporation.

14. Mohawk Sign Systems.

15. Nelson-Harkins Industries.

16. Poblocki Sign Company, LLC.

17. Seton Identification Products.

18. Supersine Company (The); Division of Stamp-Rite, Inc.

19. Vista System.

20. Vomar Products, Inc.

21. Mounting: Manufacturer's standard method for substrates indicated, Surface mounted to wall with adhesive or two-face tape.

Retain "Surface Finish and Applied Graphics" Subparagraph below if this information is not fully indicated on Drawings. Consider using Drawings except for the simplest signs.

Generally retain "Room-Identification Sign" Paragraph below for a system of small panel signs to designate rooms and spaces in a building.

C. Room-Identification Sign: Sign with smooth, uniform surfaces; with message and characters having uniform faces, sharp corners, and precisely formed lines and profiles; and as follows:

Generally retain "Sign-Panel Perimeter" or "Frame" Subparagraph below.

1. Sign-Panel Perimeter: Finish edges smooth.

a. Edge Condition: Square cut.

b. Corner Condition in Elevation: Square.

2. Mounting: Manufacturer's standard method for substrates indicated with adhesive or two-face tape.

3. Scope: Provide room identification signs for the following rooms:

a. Men’s restroom

b. Women’s restroom

c. Restroom

2.3 PANEL-SIGN MATERIALS

Generally retain one option in "Stainless-Steel Sheet" Paragraph below. Type 316 is more corrosion resistant and more expensive than Type 304.

A. Acrylic Sheet: ASTM D 4802, Type UVF (UV filtering).

B. Vinyl Film: UV-resistant vinyl film of nominal thickness indicated, with pressure-sensitive, permanent adhesive on back; die cut to form characters or images as indicated and suitable for exterior applications.

2.4 FABRICATION

A. General: Provide manufacturer's standard sign assemblies according to requirements indicated.

1. Provide welds and brazes behind finished surfaces without distorting or discoloring exposed side. Clean exposed welded and brazed connections of flux, and dress exposed and contact surfaces.

2. Conceal connections if possible; otherwise, locate connections where they are inconspicuous.

3. Internally brace signs for stability and for securing fasteners.

4. Provide rebates, lugs, and brackets necessary to assemble components and to attach to existing work. Drill and tap for required fasteners. Use concealed fasteners where possible; use exposed fasteners that match sign finish.

Engraved copy in "Surface-Engraved Graphics" Paragraph below does not leave raised text.

B. Surface-Engraved Graphics: Machine engrave characters and other graphic devices into panel surface indicated to produce precisely formed copy, incised to uniform depth.

1. Engraved Metal: Fill engraved graphics with manufacturer's standard baked enamel.

2. Engraved Opaque Acrylic Sheet: Fill engraved graphics with manufacturer's standard enamel.

3. Face-Engraved Clear Acrylic Sheet: Fill engraved copy with manufacturer's standard enamel. Apply manufacturer's standard opaque background color coating to back face of acrylic sheet.

4. Engraved Plastic Laminate: Engrave through exposed face ply of plastic-laminate sheet to expose contrasting core ply.

C. Subsurface-Applied Graphics: Apply graphics to back face of clear face-sheet material to produce precisely formed image. Image shall be free of rough edges.

D. Subsurface-Engraved Graphics: Reverse engrave back face of clear face-sheet material. Fill resulting copy with manufacturer's standard enamel. Apply opaque manufacturer's standard background color coating over enamel-filled copy.

E. Shop- and Subsurface-Applied Vinyl: Align vinyl film in final position and apply to surface. Firmly press film from the middle outward to obtain good bond without blisters or fishmouths.

PART 3 -  EXECUTION

3.1 INSTALLATION

Retain mounting methods in this article that coordinate with mounting requirements in sign-description paragraphs retained in Part 2.

A. General: Install signs using mounting methods indicated and according to manufacturer's written instructions.

1. Install signs level, plumb, true to line, and at locations and heights indicated, with sign surfaces free of distortion and other defects in appearance.

2. Install signs so they do not protrude or obstruct according to the accessibility standard.

3. Before installation, verify that sign surfaces are clean and free of materials or debris that would impair installation.

4. Corrosion Protection: Coat concealed surfaces of exterior aluminum in contact with grout, concrete, masonry, wood, or dissimilar metals, with a heavy coat of bituminous paint.

B. Mounting Methods:

1. Concealed Studs: Using a template, drill holes in substrate aligning with studs on back of sign. Remove loose debris from hole and substrate surface.

a. Masonry Substrates: Fill holes with adhesive. Leave recess space in hole for displaced adhesive. Place sign in position and push until flush to surface, embedding studs in holes. Temporarily support sign in position until adhesive fully sets.

b. Thin or Hollow Surfaces: Place sign in position and flush to surface, install washers and nuts on studs projecting through opposite side of surface, and tighten.

2. Projecting Studs: Using a template, drill holes in substrate aligning with studs on back of sign. Remove loose debris from hole and substrate surface.

a. Masonry Substrates: Fill holes with adhesive. Leave recess space in hole for displaced adhesive. Place spacers on studs, place sign in position, and push until spacers are pinched between sign and substrate, embedding the stud ends in holes. Temporarily support sign in position until adhesive fully sets.

b. Thin or Hollow Surfaces: Place spacers on studs, place sign in position with spacers pinched between sign and substrate, and install washers and nuts on stud ends projecting through opposite side of surface, and tighten.

3. Through Fasteners: Drill holes in substrate using predrilled holes in sign as template. Countersink holes in sign if required. Place sign in position and flush to surface. Install through fasteners and tighten.

4. Brackets: Remove loose debris from substrate surface and install backbar or bracket supports in position so that signage is correctly located and aligned.

5. Adhesive: Clean bond-breaking materials from substrate surface and remove loose debris. Apply linear beads or spots of adhesive symmetrically to back of sign and of suitable quantity to support weight of sign after cure without slippage. Keep adhesive away from edges to prevent adhesive extrusion as sign is applied and to prevent visibility of cured adhesive at sign edges. Place sign in position, and push to engage adhesive. Temporarily support sign in position until adhesive fully sets.

6. Two-Face Tape: Clean bond-breaking materials from substrate surface and remove loose debris. Apply tape strips symmetrically to back of sign and of suitable quantity to support weight of sign without slippage. Keep strips away from edges to prevent visibility at sign edges. Place sign in position, and push to engage tape adhesive.

C. Remove temporary protective coverings and strippable films as signs are installed.

END OF SECTION 101423
	PANEL SIGNAGE
	101423 - 1



