
SECTION 14560

CHUTES

PART 1 GENERAL

1.1 SUMMARY

A. Provide the following:

Linen chutes.

PART 2 PRODUCTS

2.1 MATERIALS

A. Provide the following:

Products as manufactured by Valiant Products or equal.

B. Chutes:

Type:  Linen chutes.

1. Chute Diameter: 

a. 24 inches

2. Chute Metal: Aluminum coated steel.

3. Chute Intake Door and Frame: UL labeled, stainless steel, satin finish.

4. Chute Discharge Door: Horizontal discharge, fusible link, self closing steel.

C. Accessories:

Sprinkler in shaft.

1. Vent at top of chute as recommended by Manufacturer.

END OF SECTION


CHUTES


14560-1

