STRUCTURAL INSPECTION

Date:
November 10, 2005

Type:
Residential/Commercial

RE:
Post-Hurricane Katrina Structural Inspection

Structure:
CMU red brick/vinyl siding on concrete foundation

Owner(s): 
Jim Warden

Property: 
1922 3rd St.



Slidell, La 



(985)643-3428 

History:

Mr. Warden contacted Dammon Engineering to perform a structural inspection on the referenced residence.  He was concerned that wind had caused water damage to the building he resides and works in. the water rose to 3 ft. inside of the building.

Findings:

The following items are noted on the visual inspection of the residence noted abve.

1. The northwest wall has cracks in the corners.

2. 80% of the vinyl siding on the east wall blew off.

3. Shingles on the roof are loose and some have been blown off.

4. Cracks in the CMU wall at the base of the front window.

5. Cracks in the cement slab starting at the window in the front of the building (east side) arching to the north wall.

6. Separation of the sheet rock ceiling at the tape joints.

7. Kitchen floor is un-even slanted to the east.

8. Water damage to the laundry room floor and ceiling.

Analysis:

1. Wind driven rainwater interred the building through the CMU brick and mortar. 

2. The cracks in the concrete denote settling of the foundation on the north south 

3. The kitchen ceiling and Laundry room sustained water damage from the wind driven rain.

Conclusion:

The wind driven rainwater was forced into the building through the concrete/. Sealing the concrete blocks with Thompson water deal three times a year will help prevent further water damage 

It is my professional opinion that this house is structurally sound. And has 

Sincerely,

Emmett G. (Pete) Dammon, P.E.

Louisiana License No. 8796

This inspection is limited to the apparent visual conditions of the structural components of this building.  It does not cover, nor attempts to cover, any components, items, and/or conditions which, by their nature or location, are concealed or are difficult or hazardous to inspect, or which require the moving of furniture, flooring materials, rugs, fixtures, appliances, or any component-part nailed, bolted, or screwed down or shut.  No opinions are expressed regarding conditions which could be discovered only by the disassembly of any component parts, special testing, or removal of any concealing objects.

Inspections are made under normal weather conditions, and are not opinions of the conditions of the property and/or structure which may exist under unusual weather conditions, such as, but not limited to floods, heavy rains or snows, high winds, temperature extremes, or any act of God.  Specific hazardous wastes, toxic substances, toxic mold, air and water quality, communicable diseases, asbestos, soil, environmental, radon, carbon monoxide, formaldehyde, building code and termite conditions are not included in this report unless otherwise stipulated.

This report is not a warranty or guarantee of the property inspected, but it is our opinion of its condition at the time inspected.  Our liability shall be limited to reimbursement of the total cost of inspection.


