Structural Inspection

December 12, 2007
RE:
Post-Katrina Structural Inspection

Owner(s):
Danny Sigur
Address:
46 Treasure Isle



Slidell, LA  70458

History:

The Sigur residence, along with approximately 55 other homes, was located on the peninsula of land jutting into Lake Pontchartrain, known as Treasure Isle.

All but a very few of the homes were completely demolished and taken away to complete oblivion during the storm known as Hurricane Katrina, most of the remaining were not suitable to be rebuilt.
Approximately 40 of these homes were raised so that the main floor was at or above elevation 16 ft. NGVD, while approximately 15 homes were at or about elevation 7 ft. NGVD.  These are generally 30 years old or older.

Sigur Property – 

Findings:

1. Mr. Sigur had a one story wood frame brick veneer, wood panel home with composition roof located on the lake side of North Treasure Isle Drive.  The floor elevation was approximately 8 feet, NGVD.  The home had outdoor covered areas, a dock in the lake and a walkway out to the dock.  Only the footings and some floor beams remained in their same locations, the rest of the house was blown away, with a few of the heavier items found south to south east of their original location in the house.
Nearby Property – 

1. Two dwellings near the property were left standing, but gutted and falling down.  These were both raised houses, one with steel framework and one with wood.  Even though both had their main floors at or above the flood level, they were both ravaged above the flood level, and require demolition.

2. These homes had all survived previous hurricanes that had flooding, including the homes that were not elevated. Floods from recent hurricanes had been in most of the homes on slabs and minimally raised, as this home was, with no structural damage to the house.  This home and others like it could have survived the flooding of this hurricane with less extensive damage, had it not been for the wind that preceded the water and pounded the Isle with winds of approximately 130 mph, for several hours before the flood water reached their floor at noon on August 29, 2005.  Even the elevated houses succumbed to the wind pounding them, as none of them were designed and built to stand up to the winds such as were homes in Rigolets Estates (built to the new International Building Code requirements), not one half mile away.

3. Lakeshore Estates, just northwest of Treasure Isle, also suffered flooding, but their homes were not destroyed.  They meet the new codes.
4. One homeowner from Treasure Isle reported that when he first arrived after the storm, the roof of his house was seen floating in Lake Pontchartrain, indicative of tornadic winds blowing these structures into the lake, before most of the debris being washed in shore, as the flood moved away from the lake.
Analysis:

Hurricane force winds ripped the Rigolets/Treasure Isle area before dawn on Monday, August 29, 2005, prior to the flood that ensued after daylight.  The western part of the eye of the hurricane passed over the Rigolets and Treasure Isle shortly after 10:00 a.m.  These winds out of the east and northeast were gusting and causing roofs and whole houses to shutter under relentless wind pressures of from 30 pounds per square foot to 45 pounds per square foot, during the pre-dawn and morning hours.  These gusting wind speeds caused variable wind pressures that were capable of shaking and dismantling the house, long before flooding arrived, the flood peaking about noon on Monday, according to an eyewitness in Rigolets Estates, approximately ¼ mile away and the study of the wind and water characteristics at this address, by ACCU Weather.  Wind direction during this period was first out of the east and northeast prior to the eye passing and then the northwest and west-northwest following the eye passing.

Winds on Lake Pontchartrain are confirmed by the Louisiana State Office of State Climatology to have reached 130 mph with gusts well over that speed.  With this, variable forces of over 20,000 lbs. acted on the roof, walls, windows, doors, etc., having the capability of shaking the building apart.
As discussed in the ACCU Weather report, in addition to the steady state wind, micro bursts of up to 1.35 times the steady state wind on shore, was blowing in an area where the on shore and offshore winds were mixing at different speeds and microburst, so that winds of tornadic velocity was formed on the peninsula.
This house was exposed to flood waters and tidal surges in previous hurricanes, as were many of the buildings on Treasure Isle.  Hurricane Isidor, with similar flood characteristics, did no structural damage to this house.
Hurricane surge waters had reached the shallow marshes and Lake Borgne long before reaching this area, and surges that were seen along the Mississippi Coast did not happen here, as they were dampened by traveling across long stretches of these shallow marsh areas, and were being dampened and blown to New Orleans and Chalmette by the winds out of the northeast, tending to hold the water back.
Again, the homes on the peninsula that were single story, faired the same as the two and three story buildings, and many of the taller buildings had columns and breakaway walls to guard against water surge.  There is only one answer – the wind did the major damage before the water arrived, and the water carried off the remnants.

Reports from people who rode out the hurricane in Rigolets Estates, Fort Pike, Irish Bayou, Lakeview Drive, Carr Drive and myself on Bayou Liberty, all attest to the fact that there was no great surge or wall of water that would have pushed over buildings, but instead a fast rising tide in all these locations, that followed the relentless wind.  These people also reported that the flood water did not reach 16 feet above sea level, and that they saw roofs and other parts of buildings, boat houses, etc., fly by them in their respective locations.
Conclusion:

It is my professional opinion that this house was wrecked and demolished by punishing winds that worked on the walls, roof, windows, and doors that were not constructed to present day standards for standing up to the high winds.  The loss was caused by the forces of the tornadic winds, prior to the maximum flood water moving in.  The house, dock, etc. probably went into Lake Pontchartrain, loose and heavy parts scattered to the southwest on the lawns, were then pushed inland by the flooding and wind changes.
When the higher flood waters came, they were moving fast and moved the remnants far inland, where most of the identifiable objects from Treasure Isle were found in the vicinity of Apple Pie Ridge, approximately three miles northeast of Treasure Isle.

Eyewitnesses:

1.
Mr. Leonard Brown – Rigolets Estates


Marlin Drive @ Moray Drive

Rigolets Estates

Slidell, Louisiana

2. Joe Perez

(Volunteer Fire Department)

St. Nicholas Church

Lake Catherine Hwy 90

New Orleans, Louisiana

3.
Kennard Jackley (video)


265 Carr Drive

Slidell, Louisiana
4.
Emmett G. (Pete) Dammon


34487 Torregano Rd. (Bayou Liberty)


Slidell, Louisiana

5.
Jimmy Ray Lemoine


Irish Bayou


New Orleans, Louisiana

6.
Grant Geiger

406 Carr Drive


Slidell, Louisiana
7. Glen Hood (tornado sighting)

1200 I-10 Service Rd. East


Slidell, Louisiana

Sincerely,

Emmett G. (Pete) Dammon, P.E.

Attachments:
1.  Before and after photos 
2.  Maps of affected areas west of center



3.  Hurricane Katrina/Lake Pontchartrain Synopsis


[image: image2.jpg]< MAPQUEST. = !

Wit Kifchen

CAR & j 9Nerlh Shore

KNE . g
& LA i&?h‘d‘.’ﬂf Howze Basch Fritchie Marsh
{w) R o
A@‘; 4 =alt Bayoy
LAKE
ORE~
éa%mr%i
"{ﬁ:—mu@ej
15185
Rieorets
E. ﬁ'{ P\‘E@‘g C_Hkl Fort
© 2005 MapQuest.cam, Inz, D205 NAYTER
Smi
_Samnt Tammany Corner Santa Resa Gﬁ
ht Kiln
_Bamnt Tammany Hickogy @
Maude vl River @a
Mﬂ _Gamesville
Amas Riwer %

(23}
~ '.v_> J‘
< No 0o 7""_‘WSV§ g
Bnnl:? (‘ansldc Park
Lakeshara
< G ;
TORR e, / Ansley
’ 2ol Hasksan Landing

o
sntehartrain B
CARR, vn.\\;;j/

{ﬁ

& 2005 MapQuest.com, Ing ; & 2005 Tele Atlas





This inspection is limited to the apparent visual conditions of the structural components of this building.  It does not cover, nor attempts to cover, any components, items, and/or conditions which, by their nature or location, are concealed or are difficult or hazardous to inspect, or which require the moving of furniture, flooring materials, rugs, fixtures, appliances, or any component-part nailed, bolted, or screwed down or shut.  No opinions are expressed regarding conditions which could be discovered only by the disassembly of any component parts, special testing, or removal of any concealing objects.

Inspections are made under normal weather conditions, and are not opinions of the conditions of the property and/or structure which may exist under unusual weather conditions, such as, but not limited to floods, heavy rains or snows, high winds, temperature extremes, or any act of God.  Specific hazardous wastes, toxic substances, toxic mold, air and water quality, communicable diseases, asbestos, soil, environmental, radon, carbon monoxide, formaldehyde, building code and termite conditions are not included in this report unless otherwise stipulated.

This report is not a warranty or guarantee of the property inspected, but it is our opinion of its condition at the time inspected.  Our liability shall be limited to reimbursement of the total cost of inspection.


[image: image1]