STRUCTURAL INSPECTION

Date:
April 14, 2006
Type:
Residential

RE:
Roof Inspection

Structure:
Wood frame/concrete foundation/brick façade/shingle roof
Owner(s): 
Judy Bergeron. 

Property: 
57223 Allen Rd


Slidell, La  

History:

Mrs. Bergeron contacted Dammon Engineering to perform a roof inspection on the referenced residence.  Mrs. Bergeron explained she hired a company from Hammond Louisiana to replace her roof.  Mrs. Bergeron reported that there were many difficulties with the crews sent out to perform the work. 
Findings:

A visual inspection of the residence revealed the following:

1. A new roof was installed within the last ninety days.
2. The roof valley on the home was not correctly installed.  The roof shingles were lapped over one half of the valley and the other side was lapped and trimmed, but not secured.

3. There are nails in the shingles that are exposed and unprotected by the shingle above it.

4. The boot around the power pole was sliced and wrapped around the power pole and not sealed.

5. The roof has heated sufficiently to seal the tabs.  To check for the proper number of nails would disturb the seals and possibly damage the shingles on a hot day.  I did lift a section of one shingle to determine if they were secured and it seemed to be loose.  I tried a different section and it was secured.  This leads me to believe that there are inconsistencies in the installation of the shingles.  Installation of the shingles by code requires six nails per shingle.

6. The garage overhang on the side that faces the house has a opening to the soffit area.

7. The fascia boards and drip edge on the garage were not reinstalled properly.

Conclusion:

It is my professional opinion that corrective action is necessary.  Once the repairs to the roof are complete, this dwelling will be structurally sound.

Sincerely,

Emmett G. (Pete) Dammon, P.E.

Louisiana License No. 8796

This inspection is limited to the apparent visual conditions of the structural components of this building.  It does not cover, nor attempts to cover, any components, items, and/or conditions which, by their nature or location, are concealed or are difficult or hazardous to inspect, or which require the moving of furniture, flooring materials, rugs, fixtures, appliances, or any component-part nailed, bolted, or screwed down or shut.  No opinions are expressed regarding conditions which could be discovered only by the disassembly of any component parts, special testing, or removal of any concealing objects.

Inspections are made under normal weather conditions, and are not opinions of the conditions of the property and/or structure which may exist under unusual weather conditions, such as, but not limited to floods, heavy rains or snows, high winds, temperature extremes, or any act of God.  Specific hazardous wastes, toxic substances, toxic mold, air and water quality, communicable diseases, asbestos, soil, environmental, radon, carbon monoxide, formaldehyde, building code and termite conditions are not included in this report unless otherwise stipulated.

This report is not a warranty or guarantee of the property inspected, but it is our opinion of its condition at the time inspected.  Our liability shall be limited to reimbursement of the total cost of inspection.


