DAMMON ENGINEERING, INC.

dammonengineering.com

ARCHITECTS					ENGINEERS

CONSULTING		DESIGN		STUDIES		EXPERT WITNESS

554 Old Spanish Trail				   					     985-649-5832

Slidell, LA 70458								            FAX 985-641-5950

# Structural Inspection

May 17, 2012

For: 	Jon Gustafson

Kelli Gustafson

26306 Cresent Cove Ln

Katy, Tx. 77494

Re: 	111 Doubloon Drive

Slidell, La. 70461

Construction:

Two-story, wood frame, vinyl veneer and composition shingle roof on a raised CMU conventional foundation.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the residence. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers Standards and Practices for Residential and Small Building Property Condition Surveys.  This report is not an explanation of cause, effect, or engineering.

History:

Jon Gustafson contacted Dammon Engineering to request a structural inspection of the home after a home inspection report noted that the rear and front porch columns were bowed.

Findings:

It was noted that the front and rear porch 8”x 8” hollow wood columns are bowed inward towards the house. The exterior walls of both floors of the home are the load bearing walls and cantilever out over the porch. There are no cracks in the columns indicating stress and upon trying to move them by a shaking motion no movement was noted.

Conclusion:

Although not ecstatically pleasing to the eye, they appear to be structurally sound.

Sincerely,

Emmett G. (Pete) Dammon, P.E.