554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

Fax: 985-641-5950

dammonengineering.com

dammoneng@bellsouth.net

# Structural Inspection

October 2, 2012

For: 	Mr. Earl Meyer

117 Kris Drive

Slidell, LA 70458

Re: 	117 Kris Drive

Slidell, La.

Construction:

Single-story, wood frame with brick and wood siding veneer, composition shingle roof on a conventional foundation with wood, carpet and ceramic flooring.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the residence. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers Standards and Practices for Residential and Small Building Property Condition Surveys.  This report is not an explanation of cause, effect, or engineering.

History:

Pam Puckett with The Puckett Team contacted Dammon Engineering on October 1 to request a structural inspection of the referenced home due to concerns from a home inspection report noting a crack in the exterior of the foundation.

Findings:

Upon inspection, a small hairline crack was noted in the exterior foundation of the home by the rear porch and next to the French doors.  Upon inspecting the inside master bedroom ceramic flooring, hairline cracks were also noted in this same area running to the closet. Upon inspecting the exterior foundation on the opposite side of the porch from where the crack was first noted, no signs of cracks in the foundation were noted on the opposite side.

Cracks were also noted in the foundation of the garage, these are normal hairline cracks found in most garages.

A visual inspection of the exterior of the residence did not reveal any cracks in the mortar between the bricks, nor were there signs of repair. Structures typically show signs of settlement in the sheetrock at the heads of doorframes and windows; this is due to the fact that they offer minimal structural support. During the inspection, no stress cracks were found around these areas and it did not appear that they were patched or repaired to cover-up settlement.

No signs of settling were found in the roofing structure that would typically be found if a structure were under large stresses from foundation settlement. Also no cracks were found in the ceiling and no repair work was evident to suggest previous settlement.

Conclusion:

It is important to note that all house foundations settle.  Usually everything settles together, and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc.

The cracks in the slab do not appear to have compromised the structural integrity of the slab.

It is my opinion that the structural integrity of this slab is currently sound.

Sincerely,

Emmett G. (Pete) Dammon, P.E.