DAMMON ENGINEERING, INC.

dammonengineering.com

ARCHITECTS					ENGINEERS

CONSULTING		DESIGN		STUDIES		EXPERT WITNESS

554 Old Spanish Trail				   					     985-649-5832

Slidell, LA 70458								            FAX 985-641-5950

# Structural Inspection

May 24, 2012

For: 	Darlene Doran, Homeowner

Shannon Balco, Homeowner’s Daughter

6218 Dunn Ave.

San Jose, CA 95123

Re: 	125 W. Forest Dr.

Slidell, La.

Construction:

Single-story, wood frame with brick/Hardie Board veneer and composition shingle roof on a conventional foundation

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the residence. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers Standards and Practices for Residential and Small Building Property Condition Surveys.  This report is not an explanation of cause, effect, or engineering.

History:

Ann Farmer with ReMAX contacted Dammon Engineering to request a structural inspection of the referenced 40 year old home due to remarks from a home inspection performed by Cahill Inspections LLC.  The report cited “Stair step cracks in the brick veneer” this was located below the bedroom window on the left side (South) of the residence (pg 3).  The report also cited a differential movement of the structural slab. (pg 19) It was stated that Ms. Doran was the original home owner and this residence has never flooded.  Dammon Engineering was then contacted by Shannon Balco, the daughter of the homeowner, to make arrangements for payment of this inspection/report.

Findings:

The brick veneer on the left side of the residence (South side) next to the window has developed stress cracks in the mortar between the bricks.  These cracks extend from above the window down to the slab and then into the slab below the surface.  An excavation of this exterior area was not performed due to roots against the slab.  The carpet and padding was removed to expose the concrete foundation

inside the room against the outside wall with the window.  An examination of the concrete slab found no major cracks in the foundation.  Minor tension cracks were found in the slab near the outside wall. However, a complete inspection of the slab was not possible because of a wall unit on the wall between the bedroom and the garage.  The garage held the owner’s possessions which blocked an inspection of the slab behind the wall unit.

The brick veneer on the right side of the residence (North Side) outside the kitchen has developed stress cracks in the mortar between the bricks; these cracks extend down into the slab.  The concrete at the side of the slab has exposed a portion of a structural reinforcement rod.  An inspection inside the residence found the kitchen floor to be ceramic tile without signs of cracks.

A visual inspection around the house found that tree roots have grown extensively in the soil near and next to the foundation.  Multiple walkways and patio slabs are either cracked or differentially settled with tree roots extending under these slabs.

A visual inspection of the interior of the residence found minor stress cracks in the sheetrock around some of the windows and door frames.

A visual inspection of the attic space revealed that the roof framing is in good condition without undue stresses.

Three separate differential measurements of the slab were taken with a Topcon Marksman RL-60B rotating laser.  The average of these three measurements revealed a 1.3” difference between the front right bedroom and the rear bedroom.  The living room and kitchen area revealed less than ¾” difference in elevation.

Analysis:

The foundation of this 40 year old residential structure has settled causing stress cracks in the brick veneer and tension cracks in the slab.  These cracks likely occurred during the first 5 years of the slab’s life.  It is also likely that tree roots have grown under the structural slab.  This structure has withstood several hurricanes in its 40 year life span that also added stresses to the foundation with only minor cosmetic damage.

The structural reinforcement rod that has been exposed in the slab, outside the kitchen, appears to be less than 1” from the exterior of the slab. The standard embedment for structural reinforcement is 3” of cover.

Structures typically show signs of settlement in the sheetrock at the heads of doorframes and windows; this is due to the fact that they offer minimal structural support. During the inspection minimal stress cracks were found around these areas and it did not appear that they were patched/fixed to cover-up settlement.

Another typical sign of large settlement is found in the roof structure.  During the inspection no signs were found in the roofing structure that would typically be found if a structure were under large stresses from foundation settlement.

Changes in the slab elevation are on the outside of what is normally acceptable.  The equipment used for these measurements has a 0.25 inch margin of error and the measurements were taken on top of carpet and padding.

Conclusion:

Houses and foundations all settle.  Usually everything settles together, and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc.

The stress cracks in the slab and in the brick veneer do not appear to have compromised the structural integrity of the slab.  The structural reinforcement rod that has been exposed does not appear to have compromised the structural integrity of the slab.

It is my opinion that the structural integrity of this slab is currently sound.

Although these root systems are present, they don’t need to be eradicated at this time as they are not affecting the foundation at this time.

Sincerely,

Emmett G. (Pete) Dammon, P.E.