554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

Fax: 985-641-5950

dammonengineering.com

dammoneng@bellsouth.net

# Structural Inspection

November 12, 2014

For: 	Shawn M Nowak

164 Shore Crest Circle

Carriere, MS 39426

Ref:     503 Driftwood Circle

Slidell, LA 70458

Construction:

Single-story, wood frame with brick veneer and composition shingle roof on a conventional foundation.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the residence.  No inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers Standards and Practices for Residential and Small Building Property Condition Surveys.  This report is not an explanation of cause, effect, or engineering.

History:

Shawn Nowak contacted Dammon Engineering requesting a structural inspection of the referenced home due to a home inspection report that noted the brick fascia on the right side of the home to be cracked.

Findings:

The home has a brick veneer, and on the right hand side of the garage (exterior) there is a crack in the motor and two bricks that extend from the top near the fascia and zigzags to almost the foundation.

An inspection was made on the inside of the garage wall and in the attic and nothing was noted to have cracked. The garage is carpeted so a visual inspection could not verify anything.

Analysis:

There appears to be rain water being trapped on this side of the home between the brick wall and the driveway. The gutters and down spouts are missing and allowing rain water to settle on the ground and getting under the foundation and causing the minor settlement.

Conclusion:

House foundations all settle.  Usually everything settles together, and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc.

It is my opinion that the structural integrity of this home is currently sound but will expand and contract with the ground movement and any additional standing water near the home.

Any tree with root systems that approach’s the foundation should be removed to prevent any foundation damage.  A contractor should be hired to seal the cracks in the brick wall to prevent insects from entering the home and should have gutters and down spouts that direct the water away from the home.

It is my opinion that the structural integrity of this home can be secured from future settling.

Sincerely,

Brian Mistich

PE:30187