Foundation Inspection

Tuesday, May 08, 2007

For:  

Mr. Ron Harris

1408 A Gause Blvd.

Slidell, LA 70458

Property inspected:

38285 Salt Bayou Rd.

Slidell, LA 70461

Construction:  The foundation is constructed of 32 30’ pilings driven into the ground to a depth of 18’.  These pilings are banded together using 2x12 treated lumber, doubled and bolted through the pilings using ¾’galvanized threaded steel bolts. The floor is 4” thick concrete.

History:

Mr.Harris contacted Dammon Enginering to request a foundation inspection of his property at 38285 Salt Bayou Rd.  The home was destroyed by hurricane Katrina with only the piling foundation and 2x12 banding remaining.

Scope:

This inspection is limited to visual observation of the remaining pilings and cement slab.

The pilings are in very good condition.  There are no visible cracks and only minimal warp is evident.  The concrete slab has several hairline cracks that should not affect the integrity of the foundation.

The elevation of the living area is at 15’4”.

Opinion:

There is a reasonable presumption of”structural Soundness” of the original design, which was constructed using accepted, conventional materials and methods.  It is my opinion that the above foundation is structurally sound and capable of being restored to its original design.

