554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

October 22, 2018

For: 	Robert & Alison Condon

1604 Peninsula Dr

Slidell, La.

Construction:

Two-story, wood frame, stucco veneer, with a composition shingle roof on a post and pier foundation.  The home is located in flood zone V.

Scope:

This inspection is limited to a visual inspection of the shell of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a structural inspection of the referenced home due to the owners concerns of flood vent count and breakaway wall installation.

Findings:

The home is elevated to meet the flood elevation in this area. The home sits on top of piling supported CMU columns distributed throughout the foot print of the home.

In-between the CMU columns there are CMU breakaway walls and wood framed breakaway walls with (6) 8x8x16 flood vents.

The CMU breakaway walls were originally installed with a gap in between the CMU columns and breakaway walls to fail in the event of a storm surge, since then the owner has installed foam insulation to seal the cracks to keep insets out.

Results:

Venting;

The existing venting appears not to be adequate for this installation.

The equation for determining the amount of needed venting is 1” per 1 square foot of enclosed area.

This home has 1880 sq. ft. of enclosed area.

This home is required to 1880 sq. in. of venting.

This home has 768 sq. in. of venting in place.

This home needs 1112 sq. in. more of venting.

CMU breakaway walls;

In-between the CMU columns there are CMU breakaway walls and wood framed breakaway walls. The amount is adequate for this installation.

Conclusion:

It is important to have some type of relief of flood water pressure in the enclosed area below the homes base flood elevation. This will allow storm water to enter and exit the enclosed areas without causing a collapse of the CMU columns.

Recommendations:

Items to be addressed are as follows:

- Install 8 more vent areas measuring 8” x 16” in the CMU walls around the perimeter of the enclosed area for a total of 1880 sq. in.

Should you have any questions or if further information is required, please feel free to contact me.

Respectfully,

Brian Mistich, P.E.

See attached pictures.