554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

February 07, 2023

David Luparello

KDM Builders LLC

309 Alan Circle

Slidell, La. 70458

Ref: 113 Moray Dr.

Slidell, La. 70461

History:

Dammon Engineering was contacted by the contractor for a structural inspection of the referenced foundation and concrete columns where a home was once damaged from Hurricane Katrina in 2005.  The contractor has requested to use the existing foundation and concrete columns for a new home to be built on the existing concrete columns.

Findings:

An overall visual inspection of the concrete foundation and concrete columns were conducted and are as noted along with the bulkhead.

All of the concrete columns are in place from where the house was along with the 2x12 framing boards from piling to piling.  A majority of these concrete columns show signs of stress where hit with debris.   Some concrete columns are missing chunks of concrete and the rebar is visible along with stress cracks.

The 2x12 framing boards are in disarray along with the Simpson Strong Tie, tie downs into the concrete columns. The boat house and boat launch bulkhead are also in disarray.

Recommendation:

Due to the new finished floor elevations of FEMA and the fact that the existing concrete columns are not structurally sound and not suitable for future use, we recommend the removal of all the concrete columns along with the foundation and install new for a new home. We also recommend a new bulkhead.

Sincerely,

Brian Mistich, P.E.