554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

December 13, 2022

T Markoff

128 Kempsey

Slidell, La. 70458

Construction:

One-story, wood frame, brick veneer, ceramic and wood flooring with a composition shingle roof on a raised conventional foundation.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a structural inspection of the referenced home due to it being elevated.

Findings:

The home has been elevated. This was accomplished by concrete blocks on a new foundation. The exterior of the concrete blocks are coated using a surface bonding cement to create a concrete finish.

It was noted in some areas of the exterior finish that the surface bonding finish has cracked due to expansion and contraction.

The interior of the home does not reveal any structural problems.

Conclusion:

It is important to note that all foundations settle.  There are no gutters on this home to properly divert the rain water away from the foundation. Usually, everything settles together and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, substandard design, tree roots, improper drainage, etc.

Recommendation:

The cracks in the exterior surface finish do not appear to have compromised the structural integrity of the home.  It appears that the structural integrity of this home is currently sound. The foundation will expand and contract with the ground movement and any standing water.

Items to be addressed are as follows:

1)  Seal the cracks in the concrete finish with a non-shrinking grout.

2) Gutters and down spouts should be added to allow proper drainage of rainwater away from the foundation.

Sincerely,

Brian Mistich, P.E.