554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

November 14, 2018

For:     Rick Hite

VIP Properties LLC

3009 Pontchartrain Drive

Slidell, La. 70458

Ref:    1537 Eastridge, Slidell, La.

Findings:

This home is in a remodel stage and the owner found the stud wall is removed that separates the kitchen from the living room. The top plate is still intact and supports the ceiling joist; it was asked how to remove the top plate to complete a structural system.

Recommendation:

It is recommended to use a rough cut 2” x 12” board that will span the width of the opening (approximately 18 feet) and attach the ceiling joist to the new board. Also, a column will need to be created by tripling up the studs inside the wall cavity at each end.

Sincerely,

Brian Mistich, P.E.