554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

January 9, 2023

Wendy Hinton

1465 Eleventh St.

Slidell, La. 70458

Construction:

One-story, wood frame, brick veneer, ceramic/carpet and wood flooring with a composition shingle roof on a conventional foundation.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a structural inspection of the referenced home due to a home inspection noted some cracking in the brick mortar and foundation.

Findings:

An overall visual inspection of the exterior of the foundation was conducted and a few items were noted. Where the exterior brick wall meets foundation mortar was added to create a chamfered edge to shed water. The brick wall on the front left corner of the home where the bedrooms are there is a recent patch of the mortar. There is also big concrete patch on the front of the homes foundation.

The right side of the home was noted to have a molded brick wall from the rainwater bouncing back up on the wall. The interior of the home does not reveal any structural problems.

Conclusion:

It is important to note that all foundations settle.  There are no gutters on this home to properly divert the rain water away from the foundation. Usually, everything settles together and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, substandard design, tree roots, improper drainage, etc.

Recommendation:

The cracks in the exterior surface finish do not appear to have compromised the structural integrity of the home.  It appears that the structural integrity of this home is currently sound. The foundation will expand and contract with the ground movement and any standing water.

Items to be addressed are as follows:

1)  Seal the cracks in the concrete finish with a non-shrinking grout.

2) Gutters and down spouts should be added to allow proper drainage of rainwater away from the foundation.

Sincerely,

Brian Mistich, P.E.