554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

January 18-2018

For: 	Daniel & Tara Gillam

2010 Old River Road

Slidell La. 70461

Construction:

One-story, wood frame, brick veneer, ceramic and wood flooring with a composition shingle roof on a conventional foundation.

History:

Dammon Engineering was contacted for a structural inspection of the referenced home due to a potential home buyer that noted a deflection in the living room beam. This inspection is limited to the living/dining room beam of the home.

Findings:

Upon inspection, the beam is visibly deflected.  A zircon level was used to measure from the floor to bottom of beam.  The level was set up on the floor and measured at the end of each side of the beam and middle.

One side of the beam was noted to have some sheet rock damage where the dining room ceiling abuts the beam.

Conclusion:

Per IBC, Chapter 16, Table 1604.3 the maximum allowed deflection is .55” and the inspected beam deflection is one inch.

Sincerely,

Brian Mistich, P.E.