554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

November 2, 2021

Gregory Fisher

2641 – 43 College St.

Slidell, La. 70458

Construction:

One-story, wood frame, brick veneer, with a composition shingle roof on a conventional foundation.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the attic of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a structural inspection of the referenced home due to a previous fire in the attic. This home is approximately 60 years of age.

Findings:

An overall visual inspection of the interior and exterior of the home was conducted. This home has evidence of a previous fire in the attic space over the bedroom and living area.

The roof is a hip roof supported at the exterior walls.  Multiple rafters show signs of charring from a fire in that area.  There was no evidence of any rafters or ceiling joists showing any signs of being stressed or fatigued.

Conclusion:

Char that forms on the surface of wood structural members is lightweight and brittle, it has little tensile, compressive or shear strength.  Movement of any of the structural members would inevitably show signs of missing areas of char and cracks where the stress(s) would have occurred.

Recommendation:

The wood frame construction that has been exposed to a fire appears to be structurally fine. It is my opinion that the structural integrity of this home is currently sound.

Sincerely,

Brian Mistich, P.E.