554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

February 26, 2019

For: Van Brechindo

27073 Cusimano Rd

Lacombe, La. 70445

Construction:

One-story raised, wood frame, vinyl and brick siding, wood and vinyl flooring with a composition shingle roof on pier and beam foundation.  (The piers are not on a continuous grade beam). The home is approximately 50 years old.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to conduct a structural inspection of the referenced home due to elevation differences noted in the flooring.

Findings:

An overall visual inspection of the exterior and interior of the home was conducted and several items 	were noted.

Under the house it was noted to have a pier and beam foundation.  The pier(s) and beam(s) may settle 	at different rates depending on soil conditions under the house.  It was also observed that the earth 	under the house is wet and the surrounding area is draining under the home from the rear and side 	yards. There is also a tree that has grown next to the front porch and has pushed back the porch roof.

In the home it was noted to have a couple of places where the flooring/decking is rotten in the kitchen 	and laundry room. Most of the doors and windows have moved alignment caused by the home settling 	over time.

Conclusion:

It is important to note that all foundations settle.  Usually, everything settles together and the fastest 	rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement 	has exponentially slowed to where there is little danger that something will happen unless caused by 	weather, expansion & contraction, tree roots, improper drainage, etc. In the case of 27073 Cusimano 	Rd both under house soil conditions and tree roots are contributing factors to foundation stability.

Recommendation:

The pier and beam foundation will expand and contract with any ground movement aggravated by 	standing water. The soil under the home needs to remain dry so that the pier supports remain stationary 	resisting any movement.

Items to be addressed are as follows:

1) Bring in fill under the unit so water will shed away from underneath.

2) Install gutters and down spouts to subsurface drainage away from the house and out to the ditch.

3) Remove all trees and roots immediately around and underneath the house.

4) Level the home.

5) Replace all rotten wood flooring and inspect for any water line leaks within the home as well as any 	termite activity.

6) Grade the yard so that storm water drains away from the building structure.

Considering the age of the home, it appears structurally sound, but will require maintenance to 	preserve its integrity for the future.

Sincerely,

Brian Mistich, P.E.

Pictures attached: