554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

February 17, 2017

For: 	Denise Chies

1561 Pennsylvania Ave

Slidell La. 70458

Ref:     301 River Landing Road

Slidell, La. 70461

Construction:

Two-story, wood frame, brick veneer, ceramic and wood flooring with a composition shingle roof on a conventional foundation.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a structural inspection of the referenced home due to a home inspection that noted some structural and elevation deference in the media room.

Findings:

The media room was constructed above the living room and master bedroom as an added feature in the attic area. The ceiling of the living room and master bedroom was designed as a ceiling joist system only and not designed to support an additional room/load above it. The construction of this media room was improperly built and does not meet code.

The construction of this room consisted of building a knee wall directly on top of OSB plywood decking that is covering the ceiling joist below. The knee wall is supporting the exterior load bearing walls of the media room. In most areas the knee wall is not supported properly over the OSB and is allowing the floor to sag.

In other areas the floor joist of the media room doesn’t even touch the knee wall that was installed to support it. In some areas the knee wall is built on top of the strong backs that are designed only for the ceiling joist.

The new floor joist for the media room does not have any blocking between the floor joists to keep the floor joist from twisting.

Conclusion:

It was asked if some additional support could be added to shore up the problem areas. Yes it can, but you will have to remove all the knee walls and install it properly with beams and columns that can carry the load all the way down to the foundation for a proper design.

Recommendation:

It is recommended that the media room be removed and utilized as the original design as an attic or totally    rebuild it with proper construction means.

Sincerely,

Brian Mistich, P.E.

See attachments: