554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

May 23, 2018

For: 	Henry Frank Fourmaux III

5814 Edgewood

Slidell, La. 70458

Construction:

Two-story, wood frame, vinyl siding, with a composition shingle roof on a pier foundation.  Home is approximately 40 years old.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to perform a structural inspection of the referenced home due to a water leak under the home that caused damage. The owner have provided us with the home inspection report and the insurance report from at the time when he purchased the home.

Findings:

The home inspection report and insurance report did not note any problems at the time of purchase. ome of the support piers are leaning in the area that the water damaged occurred. The roof line appears to have settled in this area also.

The first area seen on the right side of the home, outside the garage, has a large crack at the top which goes down the wall into slab; but stops before entering the grade beam at ground level.  A visual inspection of the interior garage floor shows a crack in the slab that appears to start at the same location of the crack in the brick veneer.

The second area seen on the right side of the garage has no crack in the brick veneer; however, a crack appears in the slab just below the brick veneer and does travel down the grade beam below ground

level.  A visual inspection of the interior garage floor shows a crack in the slab that appears to start at the same location of the crack outside.

The third area seen at the rear, near the kitchen window and exterior water faucet, has siding installed, therefore no other visual indications are available on the exterior of the home above the slab level.  The slab, below the siding, has a crack that extends down into the grade beam.  A visual inspection of the kitchen did not show any signs; however, it would appear that the kitchen has been remodeled and painted.  Countertop and cabinets are installed in front of the area seen from outside; therefore, a visual inspection of the wall and foundation were not available.

An overall inspection of the brick veneer shows that most of the weep holes at the bottom of the brick veneer have been filled with mortar.

A visual inspection of the attic in the three areas noted above revealed that ceiling joist and roof rafter structural members appear to be original and show no signs of splintering nor do they appear to be under abnormal stress(s).

Conclusion:

It is important to note that all foundations settle.  Usually, everything settles together and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc.

Recommendation:

The cracks in the brick veneer do not appear to have compromised the structural integrity of the home.  It is my opinion that the structural integrity of this home is currently sound, but the foundation will expand and contract with the ground movement and any standing water.

Items to be addressed are as follows:

1) Add gutters and down spouts to carry rain water away from the foundation and out to the street.

2) The yard should be re-graded to allow for proper drainage of rainwater away from the foundation.

3)  Seal cracks in the brick veneer, mortar with a non-shrinking grout, to prevent insects from entering the home.

Sincerely,

Brian Mistich, P.E.