554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

June 22, 2016

For: 	Billy and Marie Yarbrough

9425 bank St Ext

Clinton, La. 70722

Ref:     86 North Court Villa Dr.

Mandeville La, 70471

Construction:

Single-story, wood frame, wood siding, ceramic, carpet and wood flooring with a composition shingle roof on a conventional foundation.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the residence. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a structural inspection of the referenced home due to visible cracks in the front and rear exterior stoop and a slope in the living room floor near the fire place.

Findings:

An overall visual inspection of the exterior of the home was conducted and found that the two exterior stoops leading into the home are cracked.

One small hair line crack in the exterior foundation in the front porch area was noted along with one in the garage area.

Upon inspection, a zip level was used to verify the floor height conditions in the living room.  The level was set in the living room at zero inches and measured to the rear of the home, there is a two inch difference noted to the right rear of the home.  It is important to note that all floor height conditions are within acceptable tolerances.

Conclusion:

It is important to note that all house foundations settle.  Usually, everything settles together and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc.

It is my opinion that the structural integrity of this residence is currently sound at the time of the inspection.

Sincerely,

Brian Mistich, P.E.