554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

May 12, 2017

For: 	Andrew & Tracy Kopfinger

278 Forest Brook Blvd.

Mandeville, La. 70448

Ref:    1296 Bluff Drive

Slidell, La.

Construction:

Two-story, wood frame, brick veneer, ceramic and wood flooring with a composition shingle roof on a post-tension foundation.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a structural inspection of the referenced home. The potential home buyer found a hair line crack in the ceramic tile located in the kitchen area.

Findings:

Upon inspection, the hairline crack in the ceramic tile was noted in the kitchen area. It starts from the refrigerator continues in front of the island and under the kitchen cabinets all the way to the outside of the homes foundation.  On the outside of the home the hairline crack is visible in the foundation and appears to stop several inches down.  In the kitchen area/hallway there is a sheetrock seam above the cased opening door.

Two other areas were noted to have cracked also.  The rear ground floor porch outside the kitchen area was noted to have cracked where it meets the foundation.  Secondly the rear second floor concrete porch was noted to have a few hair line cracks in it as well. No cracks in the brick veneer, sheet rock (other than noted) or molding.

Conclusion:

It is important to note that all foundations settle.  Usually, everything settles together and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc.

Recommendation:

The hairline cracks that were noted in the report do not appear to have compromised the structural integrity of the home.  This home is currently sound. The foundation will expand and contract with the ground movement and normal settling. Tile is particularly susceptible to movement, but overall the structure appears satisfactory.  Should cracks expand or propagate in the future to any sizable extent, a  re-examination of the subject should be readdressed.

Items to be addressed are as follows:

1)  Seal the cracks in the foundation and patios with a non-shrinking grout, to prevent insects from entering the home.

Sincerely,

Brian Mistich, P.E.