554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

Fax: 985-641-5950

dammonengineering.com

dammoneng@bellsouth.net

# Structural Inspection

November 25, 2013

For: 	Mr. Raymond Williams, Jr.

Subject:	Structural Inspection of Residential Slab

200 Legendre Dr.

Slidell, LA 70458

Construction:

One and one-half-story wood-framed single family home, the structure was constructed on a conventional slab on grade then raised and supported on concrete masonry unit (CMU) walls with interior steel columns and steel beams.

Scope:

This inspection is limited to a visual inspection of the slab.  No inspection of the existing wood frame structure or mechanical or electrical systems was performed.  This report is to identify whether the raised slab foundation can structurally support the 2 story residential structure as depicted on the drawings by Architect Mr. James Macaluso, see attached.  The drawings are dated 01/12/12..

History:

Mr. Williams contacted Dammon Engineering requesting an inspection of the referenced residential slab to determine its structural condition and whether it can support the structure as shown on the drawings.

Findings:

Upon inspection, the raised concrete slab and the support system of the residence are not damaged.  No visible signs of deformation or warping were observed to the structural steel that was added to support the residence after it was raised.

The original interior concrete beam that was designed to support the original interior load-bearing wall that runs northwest to southeast was not poured under the interior load-bearing wall, but 5 feet east of that wall.  During the raising of the residence, steel beams with pipe columns were added to support this area .

Analysis:

The reference drawings show that the new 1st floor load bearing walls are to be rebuilt in the same position where they were originally built.  The additional steel beams with pipe columns under the northwest to southeast wall will support the load-bearing wall on the 1st floor as well as the loads from the 2nd floor, however will not support the additional loads and the roof system.

The reference drawings also show that some of the new 2nd floor walls do not sit directly over the 1st floor load-bearing walls.

Recommendations/Conclusion:

The roof shall be supported only on the exterior walls, therefore transferring the roof load to the exterior concrete beams.  The roof trusses shall be pre-engineered trusses and not stick built to ensure that no interior walls support the roof system or wind loads from the roof.

Additional support should be added to support the 2nd floor walls by doubling up on the 2x8 floor joist and ensuring that these double joists are directly under the 2nd floor wall(s) when running parallel.

Summary:  This slab can support the structure as proposed on the reference drawings as long as the pre-engineered roof trusses distribute the weight of the roof to the exterior walls.

Sincerely,

Brian Mistich, P.E.

3 attachments