STRUCTURAL INSPECTION

Date:
October 26, 2004

Type:
Residential

For:
Billy R Kennedy

             Pamela A Jenkins

1543 Cuttysark Cove

Slidell, La. 70458

Phone/Fax # 985-781-0024

History:

Dammon Engineering was contacted by Mrs. Pamela Jenkins to inspect the above home for structural problems.  The home is approximately 5 years old.  It is constructed of wood frame/brick veneer with composition shingle roof and vinyl fascia and soffit.

Findings:

1. It was noted that there are several hairline cracks in the foundation. These areas are visible in the front dining room of the home and the garage area. The hairline cracks in the front dinning room are approximately 11 feet in length. These cracks are notable in four pieces of ceramic tile also. 

2. It was noted that one piece of glass was cracked above the bedroom door (transom) that exited the rear of the home. 

3. No cracks were noted in the sheet rock or the brick veneer of the home.

Analysis:

1. The cracks that were noted are indicative of shrinkage and settlement over a period of time. All the underground plumbing, doors and windows appeared to be functioning properly. The cracks do not appear to be a structural problem.  

Conclusion:

All foundations at some point and time will have hairline cracks. All buildings and building materials expand and contract.  Over the time period since the house was built, it has followed a normal pattern.

The findings noted are typical of a home.  It is my professional opinion that the house is structurally sound.

Sincerely,

Emmett G. (Pete) Dammon, P.E.

Louisiana License No. 8796

This inspection is limited to the apparent visual conditions of the structural components of this building.  It does not cover, nor attempts to cover, any components, items, and/or conditions which, by their nature or location, are concealed or are difficult or hazardous to inspect, or which require the moving of furniture, flooring materials, rugs, fixtures, appliances, or any component-part nailed, bolted, or screwed down or shut.  No opinions are expressed regarding conditions which could be discovered only by the disassembly of any component parts, special testing, or removal of any concealing objects.

Inspections are made under normal weather conditions, and are not opinions of the conditions of the property and/or structure which may exist under unusual weather conditions, such as, but not limited to floods, heavy rains or snows, high winds, temperature extremes, or any act of God.  Specific hazardous wastes, toxic substances, toxic mold, air and water quality, communicable diseases, asbestos, soil, environmental, radon, carbon monoxide, formaldehyde, building code and termite conditions are not included in this report unless otherwise stipulated.

This report is not a warranty or guarantee of the property inspected, but it is our opinion of its condition at the time inspected.  Our liability shall be limited to reimbursement of the total cost of inspection.


