DESIGN ANALYSIS

CAMELLIA GARDENS

FAIR HOUSING ACCESSIBILITY GUIDELINES

Chapter  I - Accessible Building Entrance on an accessible route.


Camellia Gardens buildings will have enclosed walkways between buildings that will be level with the floor of the buildings.  Entrances to building from the exterior will include:

· Three-Foot wide entrance doors.

· Exterior landings that are level with bldg. floors and with thresholds that are less than ½” high.

· The main building entrance includes a handicap drop off and pick-up area and handicap parking at various areas around the building that are van accessible, with curb ramps that comply with ADA.

· All units have three ft wide entrance doors.  Twenty-eight units are fully handicap assessable.

Chapter II - Accessible and usable public and common use areas.


        

· Side walks and approaches to buildings and ramps at handicap parking spots will be ramped at no greater slope than 1:12.

· Common areas such as dinning area and activity rooms will have accessible toilets, three ft wide doors etc,. All doors will have the ADA required 1.5 ft side wall clearance for opening door from a wheelchair.

Chapter III -  Usable doors.

· All passage doors in all units will be three ft doors with greater than the two foot eight inch required clearance for ADA.  Closet and pantry doors do not necessarily meet this requirement

Chapter IV – Accessible route into and through the covered unit.

· All hallway, entrance and exit doors are a minimum of thirty-six inches wide with over two ft-eight inches clear opening.

· Hallways are continuous to exit doors, with no greater distance to travel to an exit than allowed by code.

Chapter V - Light switches, electrical outlets, thermostats and other                                     

 
        environmental controls.

· Switches and receptacles.  Switches will be located on walls and normally at doorways for maximum comfort in locating and operating.  Outlets will be located on walls 15 inches above floor.  These will be installed to meet N.E.C. requirements.

· Thermostats will be located on walls in an area not normally used for furniture, to eliminate possibility of blocking air movement and for operating by occupant. 

Chapter VI  -  Reinforcement for grab bars. 

Drawing A-C shows dimensions to grab bars and reinforcement.

Chapter VII  - Usable Kitchens and baths.

Twenty-eight units are fully handicap accessible for kitchens and baths.

PAGE  
2

