Sarbjit Singh Building

78492 Lee Road

Covington, Louisiana

Final Agreement from Addenda Items

1. There will be no work to install gas pumps, islands, canopy for pumps, or electrical work that is associated with the fuel sales.

2. Remove and dispose of three (3) existing tanks and pumps (2-4,000 gal. and 1-8,000 gal.)

3. No grease trap

4. No new culverts

5. The wall between store and warehouse, store and buildout, and between warehouse and buildout, will be 1-hour to roof, with no openings.

6. The walk-in cooler will be installed during the construction period by others.  McMath will furnish electrical wiring and conduit, and coordinate as required.

7. Do not provide “gas line for future use.”

8. Allowance for brick = $250/thousand

9. Allowance for canopy = $30/lin. ft.

10. Furnish and install four (4), 500 watt HPS fixtures on 25-ft. high standards, per addendum dated May 4th, 2001.

11. Wastewater treatment plant will be 1 each 1,000 gallon unit with chlorinator and effluent reduction system.

12. Quarry tile or Dial tile (Vitre Stone) will be installed per finish schedule and addendum dated May 7th 2001, with allowance of $6.00/ft., installed.

13. Resize electrical service to a 400-amp three-phase system and install panels and circuits per the drawings and addendums.

14. Concrete driveway not to be extended toward rear of building farther than front end of building on south side.  Concrete drive on North side to extend to rear and then south to a point 10 ft. beyond grocery warehouse/storage door.

