Addendum #1

Construct Parish Barn

February 20, 2001

To:
All Holders of Project Plans

RE:
Fritchie Barn

I.
Work described in the attached fire marshal letter is part of this contract.

II.
The Following changes are made to the contract drawings.

A.       Specifications, 

      1.
Section 01010, Summary of Work

1.2
Existing Conditions

Delete words “Brewster Road in Madisonville, LA”.
Delete all references to two facilities.

1.3
Driveways are in this contract.

Contract agreement for this job

2. 02.25, Standard Form of Proposal, enclosed

B.
Dwg. C-1

1.
Change T.O.S. elevation @ new bldg. to 22.5 ft.

2.
Sewage pipe for new building to treatment plant & from treatment plant to ditch shall be 4” PVC, schedule 40

3. Enclosed is a pipe bedding detail

C.
Dwg. A-1


1.
Reception office window @ stairs shall be 2/0x2/0 fixed wire glass.

2.
Cabinets in general office shall be laminated plywood, kitchen grade.

3.
2nd floor office wall & 2nd floor equipment room walls (except for exterior walls) shall be 1 hour rated 5/8” sheet rock, each side.

4.
2nd floor doors to office & equipment room shall be mark 1’s with no lite.

5.
Exterior wall of office & mechanical room on 2nd floor shall match exterior of first floor & building.

6.
2nd floor office and mechanical room shall have 2x6 ceiling joist @ 24” O.C.  Cover with 5/8” sheet rock and ½” for 1 hour exterior plywood.

D.
Stairs and Landing

Stairs shall be metal as called for on drawings. Top landing shall be wood, with 2x12’s cantilevered out to form landing. Landing rails shall be wood frame and metal siding, to match 3’-6” high railing at loft. Gates shall be 

1 ½” pipe with wire (diamond) mesh cover. Supports for gates shall be

1 ½” pipe. Install 4” pipe columns below each end of landing, with ¼”x4”x8” plate, bolted to concrete floor with four ½”x4” redhead anchors, and to landing above with 3/8”x 4” lagbolts.

E.
Add to finish schedule:

Exterior shop floors shall have light broom finish (no sealer).

Frame of building shall have one prime coat, with two coats enamel (color brown).

F.       Dwg. A-2

1.
Skylights shall be one panel wide, by 6ft long.

G.       Dwg. A-3

1.
Vinyl covered blanket insulation shall be 3” thick.

2.
Roof shall be 26 gauge standing seam.

H.       Dwg. E-2

1.
Lights in loft area shall be supported from roof trusses, using unistrut supports with jack chain.

Acknowledge receipt of addendum(s) in quote to Parish.

Acknowledge receipt of addendum(s) to Dammon Engineering, Inc, 641-5950.

Sincerely,

Emmett G (Pete) Dammon, P.E.

EGD/plc

