
SECTION 06181PRIVATE 






  GLUE LAMINATED STRUCTURAL UNITS

1 PART 1   GENERAL


.1
WORK INCLUDED



A.
Glue laminated wood beams.



B.
Preservative treatment of wood.



C.
Steel hardware and attachment brackets.


.2

seq level2 \h \r0 
WORK INSTALLED BUT FURNISHED UNDER OTHER SECTIONS




Not Used


.3

seq level2 \h \r0 
RELATED WORK



A.
Section Prefabricated steel structural supports.



B.
Framing and Sheathing.



C.
Wood Blocking and Curbing.



D.
Wood Decking.



E.
Wood Chord Metal Joists.



F.
Prefabricated Wood Trusses.



G.
Plywood Web Joists.



H.
Painting: Finishing of exposed‑to‑view surfaces.


.4

seq level2 \h \r0 
REFERENCES



A.
AITC ‑ American Institute of Timber Construction.



B.
ALSC ‑ American Lumber Standards Committee: Softwood Lumber Standards.



C.
ANSI A190.1 ‑ Structural Glued Laminated Timber.



D.
ASTM D2559 ‑ Adhesives for Structural Laminated Wood Products for Use Under Exterior (Wet Use) Exposure Conditions.



E.
AWPA ‑ American Wood Preservers' Association.


.5

seq level2 \h \r0 
SYSTEM DESCRIPTION




Not Used


.6

seq level2 \h \r0 
QUALITY ASSURANCE



A.
Manufacturer: Company specializing in manufacture of glue laminated structural units with three years minimum experience, and certified by the AITC in accordance with ANSI A190.1.



B.
Erector: Company specializing in erection of glue laminated structural units.


.7

seq level2 \h \r0 
REGULATORY REQUIREMENTS



A.
Conform to applicable code for loads, seismic zoning, and other load criteria.


.8

seq level2 \h \r0 
SUBMITTALS



A.
Submit shop drawings and product data.



B.
Indicate framing system, sizes and spacing of members, loads and cambers, bearing and anchor details, bridging and bracing and framed openings, and Submit design calculations.



C.
Provide technical data on wood preservative materials, application technique, and resultant performance information.


.9

seq level2 \h \r0 
DELIVERY, STORAGE, AND HANDLING



A.
Deliver products to site under provisions of Section 01600.



B.
Store and protect products under provisions of Section 01600.

2

seq level1 \h \r0 

seq level2 \h \r0  PART 2   PRODUCTS


.1
FABRICATION



A.
Fabricate glue laminated structural members in accordance with AITC industrial grade.



B.
Verify dimensions and site conditions prior to fabrication.



C.
Cut and fit members accurately to length to achieve tight joint fit.



seq level2 \h \r0 


A.
Fabricate member with camber built in.



B.
Do not splice or join members in locations other than that indicated, without permission.



C.
Fabricate steel hardware and connections with joints neatly fitted, welded, and ground smooth.

3

seq level1 \h \r0 

seq level2 \h \r0  PART 3   EXECUTION


.1
INSPECTION



A.
Verify that supports are ready to receive trusses.



B.
Verify sufficient end bearing area.



C.
Beginning of installation means acceptance of existing conditions.


.2

seq level2 \h \r0 
PREPARATION



A.
Coordinate placement of support items.


.3

seq level2 \h \r0 
ERECTION



A.
Set structural members level and plumb, in correct positions.



B.
Provide temporary bracing and anchorage to hold members in place until permanently secured.


.4

seq level2 \h \r0 
TOLERANCES



A.
Framing Members: 1/2 inch (12 mm) maximum from true position.


END OF SECTION
PAGE  
3

