554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

October 18, 2024

Re:  	1st Baptist Church Restroom Bldg Wall Strength

Assumptions :

Grade 60 rebar has a yield strength of 60,000 psi

Density of concrete = 150 #/ft3

Density of water (ρ)= 62.4 #/ft3

Calculate the pressure from water at depth.

The pressure from the water at the top is nothing increasing to 62.4 pcf x 5.75 ft = 358.8 psf

To get an equivalent uniform distributed load 358.8 psf x 1.03 = 369.6 psf

To get a total pressure along the wall 369.6 psf x 5.75 ft = 2,125.2 plf

Then multiply by the length of each wall 16.25 lf x 2,152.2 plf for a total pressure against the wall of  34,534.5 #

A #5 vertical rebar has been imbedded in concrete foundation @ 24 inches on center and extended up to the top of the CMU wall.  Each rebar has a yield strength of 60,000 psi.  The rebar will keep the structure from collapsing.

David Dammon

Bill Kingsmill’s contact information is as follows:

William Kingsmill (kingsmillprop@bellsouth.net)

Cell: (504) 528 3838

1200 Business Highway 190, Suite 13

Covington, LA 70433

Lawrence Wm. Haik, Sr

Republic Commercial Properties, LLC

1539 Royal Palm Dr.

Slidell, LA 70458

(985) 960-7033