554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

May 4, 2018

City Of Slidell

Building Safety and Permits

Slidell, La. 70458

Attn: Greg Gordan

Ref: Pinewood Wedding Venue

Per our inspection at the above referenced site please accept this letter of approval.

- Bottom plate to foundation with 5/8” x10” anchor bolts @ 32” O.C.

- All steel building columns anchored with ¾” x 18” anchor bolts.

- 24 inch Simpson strong tie CS 16 straps every other stud on gable walls between top and bottom floors.

- 10 inch joist hangers nailed to band and are using rafter ties nailed to every rafter to top plate.

Sincerely,

Chuck Dammon

Mike serona - Mike388068@yahoo.co.us 662-288-6080