Heritage Plastics Warehouse 4

Slab to have 4” thick 4,000 psi concrete:

Temp & Shrinkage:

[7.12.2.1]  r=0.0018 for slabs with 60ksi steel. ATS=rbh = 0.0018 x 12 x 4 = 0.0864 in2  / ft of width W2.9 has 0.087 in2 when spaced 6” o.c.

6x6 W2.9 x W2.9 W.W.M.