Phone: 985-649-5832

Fax: 985-641-5950

dammonengineering.com

dammoneng@bellsouth.net

## September 13, 2012

To: 	Jay Pittman, E.I.

St. Tammany Parish

Department of Engineering

21490 Koop Dr.

Mandeville, LA 70471

REF: B&W Trucking

130 Industrial Park

Slidell, LA

Dear Mr. Pittman,

Here are the cut and fill calculations for this site. Please see the attached cut-fill diagram to see how the below figures were calculated.

Fill needed to enclose the existing ditch below new concrete driving surface = 145 yd3

Fill needed to meet the new concrete elevations in area of existing ditch = 97 yd3

Cut from the gravel yard=168 yd3

Cut from re-working the existing on site drainage ditch=78 yd3

Total cuts = 246 yd3

Total fill   = 242 yd3

SUM = 4 yd3 extra being removed from Site

The existing crushed limestone on the Site shall be used as the base material for the concrete paving. There shall be no additional fill required for the concrete paved vehicular areas.

The existing on site drainage ditch shall be reworked to allow the new concrete elevations and meet the minimum required cuts needed to achieve “No net Fill” for the Site.. The ditch shall maintain its current load capacity.

Sincerely,

Emmett G. Dammon