Note :  01 50 00  	TEMPORARY FACILITIES AND CONTROLS

2.1 MATERIALS

A. Chain-Link Fencing: Minimum 2-inches, 9-gauge, galvanized steel or aluminum, chain-link fabric fencing; minimum 6-feet high with galvanized steel pipe posts; minimum 2-3/8-inches- OD line posts and 2-7/8-inches-OD corner and pull posts, with 1-5/8-inches-OD top and bottom rails.

1. Provide concrete or galvanized steel bases for supporting posts.

2. Provide protective barriers at bases to prevent tripping by pedestrians.

B. Portable Chain-Link Fencing: Minimum 2-inches, 9-gauge, galvanized steel or aluminum, chainlink fabric fencing; minimum 6-feet high with galvanized steel pipe posts; minimum 2-3/8-inches-OD line posts and 2-7/8-inches-OD corner and pull posts, with 1-5/8-inches-OD top and bottom rails.

1. Provide concrete or galvanized steel bases for supporting posts.

2. Provide protective barriers at bases to prevent tripping by pedestrians.

Submittals

03 30 0 - 	Concrete

A. Product Data: For each type of product indicated.

B. Design Mixtures: For each concrete mixture.

C. Steel Reinforcement Shop Drawings: Placing drawings that detail fabrication, bending, and placement.

D. Formwork Shop Drawings.

22 05 00  	COMMON WORK RESULTS FOR PLUMBING

Product Data: For the following:

1. Transition fittings.

2. Dielectric fittings.

3. Mechanical sleeve seals.

4. Escutcheons.

(Each submittal shall contain a certification that no product used contains asbestos.)

22 13 13 	FACILITY SANITARY SEWERS

Product Data: For the following:

1. Special pipe fittings.

2. Backwater valves.

3. Manhole cover adjustment ring

4.  Pipe and fittings, SDR-26 PVC Pipe

5.  Cleanouts

Inspect interior of piping to determine whether line displacement or other damage has occurred.  Inspect after approximately 24 inches of backfill is in place, and again at completion of Project.

32 31 13.53  		HIGH SECURITY CHAIN LINK FENCES AND GATES

Product Data: Include construction details, material descriptions, dimensions of individual components and profiles, and finishes for chain-link fences and gates:

1. Fence and gate posts, rails, and fittings.

2. Chain-link fabric, reinforcements, and attachments.

3. Gates and hardware.

32 92 00  	TURF AND GRASSES

A. Product Data: For each type of product indicated.

B. Certification of grass seed.

C. Product certificates.

2.1 Seed

A. Grass Seed: Fresh, new-crop seed complying with AOSA's "Rules for Testing Seeds" for purity and germination tolerances.

B. Seed Species

1. Quality: State-certified seed of grass species as listed below.

2. Sun and Partial Shade:

a) St. Augustine, plugs

b) Centipede, seed