CITY OF SLIDELL PROJECT NO. 600-129
CITY HALL PARKING LOT & COURTYARD

BOUSCAREN STREET
SLIDELL LA  70458


SECTION 04510
ELECTRICAL
PART 1 -  GENERAL

1.1 RELATED DOCUMENTS

A. Drawings and general provisions of the Contract, including General and Supplementary Conditions and other Division 1 Specification Sections, apply to this Section.

1.2 GENERAL
A. This Section covers the supply and installation of underground electrical conduit and wire.
1.3 MATERIALS
A. All materials shall be supplied by the Contractor in accordance with the Drawings.
1.4 CONDUIT
A. The Contractor shall supply the conduit with all necessary couplings and fittings.  Rigid conduit for underground electrical installation shall be polyvinyl chloride pipe Schedule 80.
1.5 CONSTRUCTION
A. The conduit shall be installed by trench excavation method in accordance with the Drawings.  Underground conduit shall normally be trench excavated except where underground conduit is designated to be placed under existing pavement or surfacing structure, in which case the conduit shall be installed by the pushed conduit method.
1.6 TRENCH EXCAVATION
A. A trench shall be excavated to the depth and grade required, and a base shall be formed to provide a firm foundation of uniform density throughout the length of the trench.  The trench shall be no wider than necessary to permit satisfactory installation of the conduit and thorough compaction of the backfill material around the conduit.  The excavation shall be performed in such a manner as to cause the least possible damage to the adjacent embankment surface and other improvements.
END OF SECTION 04510
ELECTRICAL

04510 - 1

