| - Client: | - Stolthaven | - PROJECT DETAILS |
| --- | --- | --- |
| - Ref No.: | - SL47215 | - Overhead Crane Loads |
| - By: | - M Barcelona |  |
| - Date: | - 23 Jan 2023 |  |
| - Rev Level: | - A1 |  |

To aid your customer in determining if their W12 column is strong enough to carry our crane loading I am providing the max. loading based on the following parameters.

A) THE LOAD IS AT THE EXTREME END OF THE RUNWAY BEAM

B) THE CRANE SPAN IS 15'-0"

C) THE OVERALL LENGTH OF THE RUNWAY IS 36' - 0" (CENTER/CENTER OF COLUMNS)

D) THE RUNWAY BEAMS ARE W18 x 86

D) 2 METRIC TON CAPACITY AIR POWERED CRANE

E) MIN. MAX. LOAD DISTANCE TO CENTERLINE OF COLUMN OF 4'-2"

MAXIMUM COLUMN LOAD: LL: 14.02 KIPS   DL: 4.38 KIPS   TOTAL LOAD: 18.40 KIPS

MINIMUM COLUMN LOAD : LL:   1.84 KIPS   DL: 2.00 KIPS   TOTAL LOAD:   3.84 KIPS