DAMMON ENGINEERING, INC.

dammonengineering.com

ARCHITECTS					ENGINEERS

CONSULTING		DESIGN		STUDIES		EXPERT WITNESS

554 Old Spanish Trail				   					     985-649-5832

Slidell, LA 70458								            FAX 985-641-5950

# Structural Re-Inspection

March 20, 2012

For: 	Hilman & Linda Bertrand

1337 Westlawn Dr.

Slidell, Louisiana 70460

Construction:

Two-story, wood frame with brick/vinyl veneer and composition shingle roof on a conventional foundation, the home is approximately 30 years.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the residence.  No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers Standards and Practices for Residential and Small Building Property Condition Surveys.

History:

Mr. & Mrs. Bertrand contacted Dammon Engineering to request a structural re-inspection using a Zip Level to verify the differences in elevation between the foundation of the rear addition and the foundation of the existing home.

Findings:

A Zip Level was used to measure the difference in elevations from the rear addition of the home to the existing home.  It was noted that there is a -1/4” difference in elevation from rear of the existing home to the rear of the addition.  From the front of the home to the rear of the addition, there is a total different in elevation of -3/8”.  It must be noted that there was some difficulty in locating a level surface on which to engage the zip level, due to the irregular surface of the existing floor tiles.

Conclusion:

In talking to Mr. Bertrand on March 20, 2012, he indicated that the addition was built over an existing patio slab back in the 1970’s, but that since Hurricane Katrina he has had settling problems with the addition.  Mr. Bertrand also stated that the two foundations were not tied into each other by means of dowels, which would have held the two foundations together and minimized differential settlement/shifting.

Houses and foundations all settle.  Usually everything settles together, and the fastest rate of settlement occurs in the first year after construction.  The house continues to settle and usually by the tenth year this settlement is infinitesimal.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc.

In reviewing the paper work from Cable Lock, they have noted no footings, wire mesh or dowels in the new addition of the foundation next to the existing foundation. We feel that until the two are tied into each other properly this problem will continue to occur as the house settles and shifts, and as future expansion and contraction of the foundation occurs.

Sincerely,

Emmett G. (Pete) Dammon, P.E.